ASF4512MP-FLF-EC

Firmware Version 1.2.0.5

INTRODUCTION:

ASF4512MP-FLF-EC supports 12 Gigabit combo ports, which are comprised of an RJ45 and a SFP.* 10/100/1000 CU built-in associated Mini-GBIC slots

Using IP Clustering for stacking which is a method of grouping switches regardless of geographical location in order to enable centralized management access to the switches through a single management point. This technology provides some of the benefits of stackable switches without requiring direct stacking or connections between switch units. It also allows different kinds of switches to be grouped together, as long as they have the embedded clustering software.

It offers advanced administration through a user-friendly browser interface. QoS and VLAN Supports allows greater scalability and Supports for business-critical applications. Port can be trunked to create a high bandwidth pipe to the network.

It also has comprehensive network management functions, such as Spanning Tree protocol for standard bridging, SNMP, RMON and security features.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Firmware | Version | File Name |
| --- | --- | --- |
| Loader | 1.0.2.2 | ecs4810_12m_ld_V1.0.2.2.bix |
| Software | 1.2.0.1 | ecs4810_12m_v1.2.0.1.bix |

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Phase 3 |  |  |  |
| Patch Version | 1.2.0.5 | Based on V1.2.0.4 + fix ASF4512MP-FLF-EC-00529 only | 2016/1/11 |
| Patch Version | 1.2.0.4 | CC: EC_VOB ES4626F-SW-FLF-38_ASF4512MP_Phase3_branch (2015/9/23 1:00PM) | 2015/9/23 |
| Patch Version | 1.2.0.3 | CC: EC_VOB ES4626F-SW-FLF-38_ASF4512MP_Phase3_branch (2015/7/24 1:30PM) | 2015/7/24 |
| ECN Version | 1.2.0.1 | NTC test | 2014/10/02 |
| Test Version | 1.2.0.0 | NTC test | 2014/09/29 |
| Test Version | 1.1.9.0 | NTC test | 2014/08/28 |
| Phase 2 |  |  |  |
| Current Version | 1.1.4.12 | Patch version | 2014/07/29 |
| Current Version | 1.1.4.11 | Patch version | 2014/03/25 |
| Previous Version | 1.1.4.10 | Patch version | 2014/03/03 |
| Previous Version | 1.1.4.9 | Patch version | 2014/01/16 |
| Previous Version | 1.1.4.8 | Patch version | 2013/10/07 |
| Previous Version | 1.1.4.6 | Patch version | 2013/08/15 |
| Previous Version | 1.1.4.5 | Patch version | 2013/07/30 |
| Previous Version | 1.1.4.4 | Patch version | 2013/07/04 |
| Previous Version | 1.1.4.3 | Patch version | 2013/06/21 |
| Previous Version | 1.1.4.2 | Patch version | 2013/06/07 |
| Previous Version | 1.1.4.1 | Fromal release | 2013/04/26 |
| Previous Version | 1.1.3.2 | NTC test | 2013/04/15 |
| Previous Version | 1.1.2.2 | Patch version | 2012/10/19 |
| Previous Version | 1.1.2.0 | TS test | 2012/10/05 |
| Previous Version | 1.1.0.1 | Fromal release | 2012/04/20 |
| Phase 1 |  |  |  |
| Previous Version | 1.0.9.1 | NTC test | 2012/03/28 |
| Previous Version | 1.0.9.0 | NTC test | 2012/03/07 |
| Previous Version | 1.0.8.0 | Fromal release | 2011/12/20 |
| Previous Version | 1.0.7.2 | NTC test | 2011/12/08 |
| Previous Version | 1.0.7.1 | NTC test | 2011/11/10 |
| Previous Version | 1.0.6.3 | Patch version | 2011/09/22 |
| Previous Version | 1.0.6.2 | Patch version | 2011/08/15 |
| Previous Version | 1.0.6.1 | Patch version | 2011/07/01 |
| Previous Version | 1.0.6.0 | NTC test | 2011/06/03 |
| Previous Version | 1.0.3.8 | NTC test | 2011/05/06 |
| Previous Version | 1.0.3.7 | NTC test | 2011/04/15 |
| Previous Version | 1.0.3.4 | NTC test | 2011/02/25 |
| Previous Version | 1.0.3.3 | Full test | 2010/12/31 |
| Previous Version | 1.0.2.2 | Full test | 2010/10/24 |
| Previous Version | 09.21.16.13 | Internal test | 2010/09/21 |
| Previous Version | 09.10.18.08 | Internal test | 2010/09/10 |

HARDware compatibility:

ASF4512MP-FLF-EC (ECS4810-12M)

ASE3526MP-FLF-EC (ECS3810-26T)

BootPROM compatibility:

ALL

Network Management Software Support:

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Accton Product Feature List |  | ECS4810-12M |
| --- | --- | --- |
| Product Configuration |  | 12 Combo GE ports + 1 management port |
| Switch ASIC |  | Marvell 98DX3031 |
| PHY |  | Marvell 88E1340(S) + 88E1112 |
| Flash Memory |  | 32MB |
| SDRAM |  | 128MB |
| CPU |  | 98DX3031 embedded CPU |
| L2 MAC Table |  | 16K |
| PoE |  | No |
| Standalone/Stackable |  | Standalone |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 6 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk GE |
|  | Cisco EtherChannel Liked | YES 1) Total 6 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES 1) Unicast : MAC SA/DA |
| VLAN | IEEE 802.1Q Tagged Based | YES |
|  | Port-Based | YES |
|  | MAC-Based | YES |
|  | IP-Based | YES |
|  | Private VLAN | YES (Traffic Segmentation per port/VLAN by chip) |
|  | GVRP | YES |
|  | 802.1v Protocol | YES |
|  | IPv6 VLANs | YES |
|  | voice VLAN | YES |
|  | 4K (4092) VLANs | YES |
|  | VLAN Trunking | YES |
| Spanning Tree | IEEE 802.1D | YES |
|  | IEEE 802.1s | YES (32 instances) |
|  | IEEE 802.1w | YES |
|  | Spanning Tree Fast Forwarding | YES |
|  | Loopback Detection | YES (can select port disable in phase 2) |
|  | Auto Edge Port | YES |
|  | BPDU Filter/Guard | YES |
|  | Root Guard | YES |
| Broadcast/Multicast/UnknownUnicast Storm Control |  | YES (no Broadcast, B+M, B+M+DLF limit) |
| IGMP Snooping | IGMP Snooping v1/v2 | YES |
|  | IGMP v1/v2 querier support | YES |
|  | IGMP Immediate Leave | YES |
|  | IGMP Filtering/Throttling | YES |
|  | IGMP SNP Leave Proxy | YES |
|  | IGMP v1/v2/v3 Proxy | YES |
|  | IGMPV3 Snooping | YES |
| MVR |  | YES (groups - admin: 1024, active: 1024) |
| Jumbo Frame |  | 10K |
| QinQ |  | YES |
| select QinQ |  | YES |
| EAPS |  | NO |
| Auto MDI/MDIX |  | Auto |
| Types of GBIC and SFP support (LX, SX, LH, FX) |  | YES |
| G.8032 (ERPS) |  | YES (support 6 rings) |
| Y.1731 |  | YES |
| 802.1ag (CFM) |  | YES |
| 802.3ah (OAM) | CO & CPE | YES |
| QoS Features |  |  |
| Number of priority queues |  | 8 queues/port |
| Scheduling for priority queues | WRR Priority scheduling | YES (Shaped Deficit WRR) |
|  | Strict Priority scheduling | YES |
|  | Hybrid | YES (Shaped Deficit WRR + Strict) |
| COS | 802.1p based COS | YES (per system) |
|  | IP TOS Precedence based COS | NO |
|  | IP DSCP based COS | DSCP Only (per system) |
|  | TCP/UDP Port based COS | YES (no command) |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
|  | Egress | YES 1) Per Port-based 2) FE: - 64Kbps ~ 100Mbps - Resolution 64Kbpls 3) GE: - 64Kbps ~ 1000Mbps - Resolution 64Kbps |
| DiffServ |  | YES |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | YES |
| Dynamic Port Security (MAC-based) |  | YES |
| MAC Limitation per port |  | YES |
| 802.1x (authenticator) | Port-Based | YES |
|  | MAC-Based | YES |
|  | VLAN Assignment | YES |
|  | QoS Assignment (w/o ACL profile) | YES |
|  | QoS assignment w/ ACL profile | YES |
|  | Guest VLAN | YES |
|  | Supplicant Support | YES |
|  | EAPOL frames pass-through | YES |
| MAC Authentication |  | YES |
| WEB Authentication |  | YES |
| ACL | Number of rules | 1024 (extended mode 512 rules) |
|  | L2/L3/L4 | YES 1) L2 SA/DA MAC, VLAN based 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4) IPv6 |
|  | Time-Based (Time Range) | YES |
| RADIUS authentication |  | YES |
| RADIUS authorization and accounting |  | YES |
| TACACS+ Authentication |  | YES |
| TACACS+ authorizaion and accounting |  | YES |
| HTTPS and SSL (Secured Web) |  | YES |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES |
| User name password authentication | Local Authentication | YES |
|  | Remote Authentication via RADIUS/TACACS+ | YES |
| Management Interface Access Filtering | SNMP | YES |
|  | WEB | YES |
|  | TELNET/SSH | YES |
| Intrusion Lock (Link Detection) |  | YES |
| MAC filter |  | YES |
| Traffic Segmentation |  | YES |
| PPPoE Intermediate Agent |  | NO (will add in future) |
| Management Features |  |  |
| Cisco Like CLI |  | YES |
| CLI filtering and terminal setting |  | NO |
| Web Based Management |  | YES |
| Telnet | Client | YES |
|  | Server | YES (8) |
| Software Download/Upgrade | TFTP | YES |
|  | Xmodem / Ymodem | YES (Runtime, Loader) |
|  | Diag support TFTP download | NO |
|  | FTP | YES |
|  | HTTP | YES |
| Dual Image |  | YES |
| Configuration Download/Upload | TFTP | YES (Multiple Copies) |
|  | HTTP | YES (Multiple Copies) |
| Auto upgrade | TFTP | YES |
| Auto upgrade | FTP | YES |
| SNMP | v1 | YES |
|  | v2c | YES |
|  | v3 | YES |
| RMON | RMON I (1,2,3,9 group) | YES |
|  | RMON 2 | YES |
| MIB I/II | RFC 1213 | YES |
| BOOTP | Client | YES |
|  | Relay | NO |
| DHCP | Client | YES |
|  | Relay | NO |
|  | Snooping | YES |
|  | Snooping Option 82 | YES |
|  | Dynamic Provision (via Option 66,67) | YES |
|  | Option 82 | YES |
|  | Relay option 82 | YES |
|  | Server | No |
| IP Source Guard |  | YES (user can config max. binding entry) |
| Port Mirroring |  | YES |
| Remote Port Mirroring |  | YES |
| Event/Error Logging | Syslog (Local Flash) | YES 1) 512 entries 2) 256Kbyte Flash |
|  | Remotelog (RFC3164) | YES 1) Cisco-like 2) Support up to 5 Servers |
|  | SMTP (E-mail Notification) | YES |
| DNS | Client | YES |
|  | Proxy | YES |
| Remote Ping |  | YES |
| SNTP |  | YES |
| NTP |  | NO |
| IP Clustering |  | YES (36 members) |
| LLDP(802.1ab) | Link Layer Discovery Protocol | YES |
|  | LLDP-MED | YES |
|  | IEEE 802.3at | NO |
| UPnP |  | NO |
| Banner |  | YES |
| MAC Flush |  | YES |
| sFlow |  | YES |
| Dynamic ARP inspection (DAI) |  | YES |
| VLAN mirror |  | YES |
| MAC Based mirror |  | YES |
| ATC |  | YES |
| Delay reload |  | YES |
| CLI “show debug” |  | YES (DHCP,xSTP) |
| CLI “show tech” |  | YES |
| CLI “show interfaces brief” |  | YES |
| Cable Diagnostics / TDR |  | YES |
| Port Utilization (kbits/sec, Pkts/sec, % Util in recent 300secs) |  | YES |
| IPv6 Management (Telnet Server/ICMP v6) |  | YES |
| Green Ethernet |  | YES |
| multiple management ip support |  | YES |
| PoE |  | NO |
| Monitor Environment | Power Status | YES |
| IPv6 Features |  |  |
| IPv4/IPv6 Dual Protocol Stack |  | YES |
| IPv6 Address Types | Unicast | YES |
|  | Multicast | This is only used internally. Not configurable. |
| ICMPv6 |  | YES |
| ICMPv6 Redirect (Host) |  | YES |
| IPv6 Path MTU Discovery |  | YES |
| IPv6 Neighbor Discovery | Router Discovery | YES |
|  | Duplicate Address Detection | YES |
|  | Static Cache Entry | NO |
|  | Parameter Discovery | YES |
|  | Prefix Discovery | YES |
|  | Address Resolution | YES |
|  | Unreachable Neighbor Detection | YES |
|  | Redirect (Host) | YES |
| Stateless Autoconfiguration |  | YES |
| Manual Configuration |  | YES |
| Generic Prefix |  | NO |
| SNMP over IPv6 |  | YES |
| HTTP over IPv6 |  | YES |
| SSH over IPv6 |  | YES |
| IPv6 Telnet Support |  | YES |
| IPv6 DNS Resolver |  | YES |
| IPv6 RADIUS+ Support |  | NO |
| IPv6 TACACS+ Support |  | NO |
| IPv6 Syslog Support |  | YES |
| IPv6 SNTP Support |  | YES |
| IPv6 SMTP Support |  | NO |
| IPv6 TFTP Support |  | YES (CLI not support) |
| Remote IPv6 Ping |  | YES |
| MVR6 |  | YES |
| IPv6 QoS |  |  |
| IPv6 ACL | Number of rules: |  |
|  | L2/L3/L4 | YES |
| IPv6 DSCP remapping CoS |  | NO |

Layer 4 Sanity Check Filters:

- Packet is of type IPv4/6 TCP, but sent using MAC multicast mechanism

- TCP packet with all flags set to zero

- TCP packet with FIN URG and PSH flags set

- Both SYN and FIN flags in TCP packet are set

- Both SYN and RST flags in TCP packet are set

- TCP or UDP packet has source or destination port set to zero

- All the above TCP packet checks

- Fragmented ICMPv4 packet

- ARP packet MAC SA addresses in MAC header and in ARP packet payload do not match

Installation and Configuration Notes:

In general, the ASF4512MP Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ASF4512MP Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

To download new firmware form a TFTP server, enter the IP address of the TFTP server, select “opcode” as the file type, then enter the source and destination file names. To start the new firmware, enter the “reload” command or reboot the system.

CLI commands:

Console#copy tftp file

TFTP server IP address: 192.168.1.23

Choose file type:

1. config:  2. opcode:  : 2

Source file name: opcode.bix

Destination file name: opcode.bix

\Write to flash programming.

Programming flash started.

Success.

Console#reload

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V1.2.0.5, Loader 1.0.2.2

|  | EPR | Headline |
| --- | --- | --- |
| 1 | ASF4512MP-FLF-EC-00529 | SyncE didn't work on port12 |

Runtime V1.2.0.4, Loader 1.0.2.2

|  | EPR | Headline |
| --- | --- | --- |
| 1 | ASF4512MP-FLF-EC-00524 | enable SSH server and generate key cause CPU rising periodically |

Runtime V1.2.0.3, Loader 1.0.2.2

|  | EPR | Headline |
| --- | --- | --- |
| 1 | ASF4512MP-FLF-EC-00506 | OAM dying gasp occurred and clear at the same time if power off switch |
| 2 | ASF4512MP-FLF-EC-00515 | Enhancement: add Non-STP Loopback detection function |
| 3 | ASF4512MP-FLF-EC-00518 | error msg is shown when diag execute swinit |
| 4 | ASF4512MP-FLF-EC-00520 | VLAN translation on trunk port - CLI and SNMP |
| 5 | ASF4512MP-FLF-EC-00522 | refine VLAN translation |
| 6 | ASF4512MP-FLF-EC-00523 | remove setting of VLAN trasnlation from port-channel if there is no member |

Runtime V1.2.0.1 ; Loader V1.0.2.2

| ASF4512MP-FLF-EC-00517 | mac addr of DUT in uboot and runtime is different - Only update the loader to V1.0.2.2 to fix this issue |
| --- | --- |

Runtime V1.2.0.1 ; Loader V1.0.2.1

| ASF4512MP-FLF-EC-00501 | Enhancement: support new chip in loader - Only update the loader to V1.0.2.1 to fix the craft port issue. |
| --- | --- |

Runtime V1.2.0.1 ; Loader V1.0.2.0

| ASF4512MP-FLF-EC-00514 | IPConfiguration: The DUT can't get the IP when set the IP address to dhcp mode. |
| --- | --- |
| ASF4512MP-FLF-EC-00509 | IPConfiguration: The DUT can't get the IP when set the IP address to dhcp mode. |

Runtime V1.2.0.0 ; Loader V1.0.2.0

| ASF4512MP-FLF-EC-00512 | Diffserv: bundle rate doesn't work |
| --- | --- |
| ASF4512MP-FLF-EC-00511 | WEB: The link status of ports don't show the information on web page. |
| ASF4512MP-FLF-EC-00508 | ConfigurationFile: The DUT will dumping the exception message and crash when type the "copy startup-config running-config" |

Runtime V1.1.9.0 ; Loader V1.0.2.0

| ASF4512MP-FLF-EC-00465 | Error messages and swith reboots by itself during uploading F/W |
| --- | --- |
| ASF4512MP-FLF-EC-00503 | Enhancement: Add squashfs compress function for runtime image |
| ASF4512MP-FLF-EC-00498 | MAC ACL support 802.1p(COS) field |
| ASF4512MP-FLF-EC-00499 | Change Priority of PPPoE and MAC ACL |
| ASF4512MP-FLF-EC-00488 | Failed to set strict queue mode. |
| ASF4512MP-FLF-EC-00487 | [SYNCEDRV] Show I2C error message on board that does not support SyncE |
| ASF4512MP-FLF-EC-00478 | Phenomenon: Switch CPU utilization 100% and web fails after OPENSSL vulnerability test. |
| ASF4512MP-FLF-EC-00477 | Disable igmp snooping on specific vlan then multicast traffic will not flood in that vlan |
| ASF4512MP-FLF-EC-00475 | capability [web]: web display error |
| ASF4512MP-FLF-EC-00456 | ECS4810-12M MIB issue |
| ASF4512MP-FLF-EC-00492 | EEnhancement: Add command for un-tagged frame type |

Runtime V1.1.4.12 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00500 | I2C: Still receive 1 byte data even SFP return NACK in READ mode, to work around with Source Photonics SPL-45GB-EBZ-CDA |
| --- | --- |
| ASF4512MP-FLF-EC-00495 | VLAN Translation not work for multicast |
| ASF4512MP-FLF-EC-00476 | VLAN 4093 will disappear in mst configuration when move it from instance 0 to another instance |

Runtime V1.1.4.11 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00472 | trap of alarm input is sent when i2c bus fails |
| --- | --- |

Runtime V1.1.4.10 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00461 | CPU process increase to 100% |
| --- | --- |
| ASF4512MP-FLF-EC-00462 | web GUI problem : ERPS clear command |
| ASF4512MP-FLF-EC-00463 | web problem : The dhcp of vlan 1 could not disabled from GUI. |
| ASF4512MP-FLF-EC-00464 | web problem : can not remove vlan 1 on trunk port |
| ASF4512MP-FLF-EC-00468 | edit error in config file manually and copy to startup-config will reset switch to factory default |
| ASF4512MP-FLF-EC-00471 | one mvr source port receives only 1 query, but forward 2 querys to another mvr souce port |
| ASF4512MP-FLF-EC-00473 | mvr proxy switching sends igmp report with source ip 0.0.0.0 |
| ASF4512MP-FLF-EC-00466 | Enhancement: Add Daylight Saving Time (DST) function |

Runtime V1.1.4.9 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00467 | Enhancement: add Non-STP Loopback detection function |
| --- | --- |
| ASF4512MP-FLF-EC-00460 | MAC FLASH of ERPSv1 and v2 |
| ASF4512MP-FLF-EC-00458 | CFM Cofiguraton lost after reboot device. |
| ASF4512MP-FLF-EC-00457 | Enhancement: OAM support dying gasp. 1. dying gasp clear OAM event log 2. linkup/down OAM event log 3. clear OAM event log CLI command |
| ASF4512MP-FLF-EC-00455 | STP loopback detection "shutdown" works incorrectly |

Runtime V1.1.4.8 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00453 | Dos-Protection will drop multicast traffic which src/dst port is zero.Need command to disable/enable Dos-Protection function. |
| --- | --- |
|  | it doesn't rewrite new messages to memory neither send them to remote syslog server |
|  | Enhancement: ignore the fake link up/down in power saving mode |

Runtime V1.1.4.6 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00450 | Log : There are no syslogs from DUT to the (IPv4/IPv6) host . |
| --- | --- |

Runtime V1.1.4.5 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00448 | When we create a static port-channel without any member, we can't see this setting via web. |
| --- | --- |
|  | ATC_ allow to enable ATC BC and Storm control MC |
|  | NTP cause CPU alarm time counter error |

Runtime V1.1.4.4 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00449 | Clock Source Selectiong, number lower shall be better |
| --- | --- |
| ASF4512MP-FLF-EC-00447 | erps forced-switch won't trigger "renew" mac-address table |
| Sync-E | Change the DPLL operation mode of SyncE ASIC as Automatic Normal Mode when using manual clock source selection mode to improve the output clock performance. Per Zarlink FAE’s suggestion, the output clock jitter would be 0ppm when it enters Holdover Mode. |

Runtime V1.1.4.3 ; Loader V1.0.0.8

|  | Update correct file for sync-e which is wrong in V1.1.4.2 |
| --- | --- |

Runtime V1.1.4.2 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00443 | Sync-e: active source clock status always shows bad |
| --- | --- |
| ASF4512MP-FLF-EC-00440 | when igmpsnp group entry is full, configure more static groups will cause exception sometimes |
| ASF4512MP-FLF-EC-00441 | mldsnp cannot filter group report according to profile configuration |
| ASF4512MP-FLF-EC-00442 | SSM packet will cause port link down and up |
| ASF4512MP-FLF-EC-00444 | CLI: remove control-plane from configure, because it's useless in this project |

Runtime V1.1.4.1 ; Loader V1.0.0.8

| ASF4512MP-FLF-EC-00438 | IGMP-Auth: Continuous authentication fail, console print error message and device auto reboot. |
| --- | --- |
| ASF4512MP-FLF-EC-00435 | SNMP: Can't compile MIB file successful. |

Runtime V1.1.3.2 ; Loader V1.0.0.8

|  | [enhancement] IGMP authentication |
| --- | --- |
|  | [enhancement] need to be able to filter the PVST packets of Cisco |
| ASF4512MP-FLF-EC-00432 | Web UI: front panel display wrong linkup port (copper -> fiber) |
| ASF4512MP-FLF-EC-00430 | [MLDSNP] "no immediate-leave by-host-ip" should not shown in runnug-config since it is default configuration |
| ASF4512MP-FLF-EC-00428 | Enhancement: Rule ctrl does not use ace type from rule_type.h |
| ASF4512MP-FLF-EC-00413 | Enhancement: MIB file modification to meet standard specification (ipcfg/rip/ospf ,etc.) |
| ASF4512MP-FLF-EC-00403 | Enhancement: max active ports of trunk |
| ASF4512MP-FLF-EC-00407 | CFM: Range of MA name is wrong in web help message page. |
| ASF4512MP-FLF-EC-00429 | CLI command "Show interface transceiver" fails after the SFP has been inserted for more than 248 days |
| ASF4512MP-FLF-EC-00424 | packets cant be forwarded if injecting traffic to fiber ports during switch booting up |
| ASF4512MP-FLF-EC-00426 | show igmpsnp group exception when port switch to another vlan with multicast data injecting |
| ASF4512MP-FLF-EC-00414 | copy startup-config tftp (copy startup-config ftp) ---> Destination file name only allow 32 characters |
| ASF4512MP-FLF-EC-00423 | Can’t set default gatewat for craft port |
| ASF4512MP-FLF-EC-00410 | Management port link up/down status doesn't has syslog and trap and web UI change |
| ASF4512MP-FLF-EC-00421 | mac-vlan setting can't be saved |
| ASF4512MP-FLF-EC-00404 | Configure file on switch with name "p", then it could never be deleted. |
| ASF4512MP-FLF-EC-00425 | [Netinsight]Terminal length and width commands didn't work well |

Runtime V1.1.2.2 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00420 | mac-vlan configure is lost after reload |
| --- | --- |
| ASF4512MP-FLF-EC-00419 | Enable dhcp snooping client can't get IP |

Runtime V1.1.2.0 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00396 | Enhancement: AGG [TC-15(c)], send OAM critical event and log when port is shutdown by user |
| --- | --- |
| ASF4512MP-FLF-EC-00395 | Enhancement: AGG [TC-08], user can specify data pattern and priority for CFM Loopback test. |
| ASF4512MP-FLF-EC-00415 | Enhancement: add vlan translation function |
| ASF4512MP-FLF-EC-00416 | Enhancement: add show debug function for DHCP |
| ASF4512MP-FLF-EC-00417 | Enhancement: add show debug function for XSTP |
| ASF4512MP-FLF-EC-00409 | Redundant power status is not correct |
| ASF4512MP-FLF-EC-00411 | After switch reload, GUI port status failed |
| ASF4512MP-FLF-EC-00402 | SFP Port Shutdown need to TX disable |
| ASF4512MP-FLF-EC-00405 | enable MDLv2 Snooping, RFC4541 |
| ASF4512MP-FLF-EC-00398 | LLDP: Can't display remote vlan name when length over 32. |
| ASF4512MP-FLF-EC-00400 | Trap: currentAlarmWarnCease(25) doesn't connect to correct DDM log message |
| ASF4512MP-FLF-EC-00401 | MIB: MIB OID is not consistent with superset |

Runtime V1.1.0.1 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00387 | This issue should be caused by specific DDM transceiver. If default value of DDM is lower or higher than setting value. |
| --- | --- |
| ASF4512MP-FLF-EC-00400 | Trap: currentAlarmWarnCease(25) doesn't connect to correct DDM log message |

Runtime V1.0.9.1 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00389 | modify the DDM command interface such as no or disable transceiver-threshold xxx |
| --- | --- |
| ASF4512MP-FLF-EC-00287 | LLDP: Web page can't display remote multiple device in detail. |
| ASF4512MP-FLF-EC-00375 | Project specific backdoor passwd |
| ASF4512MP-FLF-EC-00328 | WEB: update help files for new functions |
| ASF4512MP-FLF-EC-00394 | ARP: ARP entry on Craft port can not be deleted immediately by clear arp-cache. |
| ASF4512MP-FLF-EC-00391 | Failed to clear arp cache on craft port |
| ASF4512MP-FLF-EC-00393 | Exception: The device occrued exception error and WebUI crashed. |
| ASF4512MP-FLF-EC-00372 | Enhancement: MIB file modification to meet standard specification |
| ASF4512MP-FLF-EC-00220 | LLDP: Can't display LLDP port information details on web |

Runtime V1.0.9.0 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00376 | [CODESONAR] Analysis for OAM |
| --- | --- |
| ASF4512MP-FLF-EC-00380 | ERPS: IGMP control packer from trunk member can not be blocked when the trunk is set as a ring port. |
| ASF4512MP-FLF-EC-00385 | Can't apply minus transcevier-treshold , if I didn't insert DDM transceiver |
| ASF4512MP-FLF-EC-00388 | Enhancement: add software watchdog function |
| ASF4512MP-FLF-EC-00341 | ECS4810-12M When clicking the action button, sometimes this page is displayed : 404 Not found |
| ASF4512MP-FLF-EC-00323 | RSPAN: Session range on WebUI is wrong. |
| ASF4512MP-FLF-EC-00285 | ACL: If one ACE includes source port and destination port, this ACE cann't be delete from WebUI. |
| ASF4512MP-FLF-EC-00381 | Syslog: Coding for memset on SYSLOG_MGR_AddFormatMsgEntry_Smtp |
| ASF4512MP-FLF-EC-00373 | [Enhancement] craft port display VLAN 4097 interface when show ARP command, it should use craft |
| ASF4512MP-FLF-EC-00333 | Enhancement: add command to display ARP statistic |
| ASF4512MP-FLF-EC-00386 | show tech-support command on v1.0.8.0 do not respond any information. |
| ASF4512MP-FLF-EC-00384 | sflow: restart sflow without destination IP, the switch must froze displaying some debug message |
| ASF4512MP-FLF-EC-00379 | Trap: modify SFP threshold trap to correct value |

Runtime V1.0.8.0 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00378 | Syslog: not support DDM still trigger syslog for DDM alarm/warning |
| --- | --- |
| ASF4512MP-FLF-EC-00372 | Enhancement: MIB file modification to meet standard specification |

Runtime V1.0.7.2 ; Loader V1.0.0.7

| ASF4512MP-FLF-EC-00296 | SelectiveQinQ: SelectiveQinQ configurataion does not be implememt on WebUI. |
| --- | --- |
| ASF4512MP-FLF-EC-00374 | CLI: revise epld version for show version |
| ASF4512MP-FLF-EC-00363 | System: When upgrade image; all command session timeout, IGMP groups losing and print error message. |
| ASF4512MP-FLF-EC-00362 | System: When upgrade image; all command session timeout, IGMP groups losing and print error message. |
| ASF4512MP-FLF-EC-00283 | switch freeze |
| ASF4512MP-FLF-EC-00112 | PortSecurity: DUT not discard invalid packets as configure maximum counters. |
| ASF4512MP-FLF-EC-00364 | System: DUT should not have intrusion MAC address and send Trap as learning address less maximum counter. |
| ASF4512MP-FLF-EC-00358 | HTTPs: the CN in the HTTPs certification is incorrect |
| ASF4512MP-FLF-EC-00370 | System: The unicast packets always are flooded after reboot the DUT. |
| ASF4512MP-FLF-EC-00361 | System: After reboot the DUT; unicast traffic flooded, all command session timeout and LACP trunk not produced. |
| ASF4512MP-FLF-EC-00366 | SNMP: "swProdDescription" value is not correct |
| ASF4512MP-FLF-EC-00360 | ECS4810-12M Trunk problem, The traffic almost never re-route if one of the Ethernet links are removed so the trunk only consi |
| ASF4512MP-FLF-EC-00354 | ECS4810-12M port-channel load balance issue |
| ASF4512MP-FLF-EC-00369 | System: Shutdown and no shutdown port, DUT cannot send ICMP-Reply. |
| ASF4512MP-FLF-EC-00367 | SNMPv3: The DUT can't send traps correctly with 2 subnet received hosts |
| ASF4512MP-FLF-EC-00357 | System: The CCMs is looped in system as receiving port is MIP. |
| ASF4512MP-FLF-EC-00359 | SNMP: undefined trap - ecs4810-12mTrapsPrefix.140 |
| ASF4512MP-FLF-EC-00355 | HTTP: launching HTTP with DOS prompt window causes unexpect message |
| ASF4512MP-FLF-EC-00365 | System: CLI return error message as topology change. |
| ASF4512MP-FLF-EC-00356 | HTTPs: launching HTTPs service causes unexpected messages displayed on the CLI |
| ASF4512MP-FLF-EC-00351 | [MIB File Issue] ECS4810-12M MIB does not define trap No.140 |
| ASF4512MP-FLF-EC-00332 | SNMP:Can't getbulk from Object Label = amtrMacAddrAgingStatus(Object ID = 1.3.6.1.4.1.1991.1.16.1.1.1.8.3) |
| ASF4512MP-FLF-EC-00274 | [ACL] Even if TOS number of the packet match permit entry in ACL rule, the packet can't be forwarded by DUT |
| ASF4512MP-FLF-EC-00174 | TFTP: TFTP transmission via craft port fail |
| ASF4512MP-FLF-EC-00279 | SNMP_qBridgeMIB: dot1vProtocolGroupTable & dot1vProtocolPortTable are no use. |

Runtime V1.0.7.1 ; Loader V1.0.0.6

| ASF4512MP-FLF-EC-00343 | Enhancement: When dual firmware master is failed, boot from backup firmware |
| --- | --- |
| ASF4512MP-FLF-EC-00344 | Enhancement: Support DDM in MIB node |
| ASF4512MP-FLF-EC-00088 | CFM: Use port mac for up MEP. |
| ASF4512MP-FLF-EC-00348 | Enhancement: ERPS statistics information |
| ASF4512MP-FLF-EC-00345 | Enhancement: ERPS work with link aggregation on the ring ports |
| ASF4512MP-FLF-EC-00352 | ECS4810-12M Add detail trap information on log message |
| ASF4512MP-FLF-EC-00246 | ATC: ATC does not be implemented on WebUI. |
| ASF4512MP-FLF-EC-00302 | correction to grammar/spelling |
| ASF4512MP-FLF-EC-00141 | SNMP: MIB browser get error when MIB walk by craft port |
| ASF4512MP-FLF-EC-00296 | SelectiveQinQ: SelectiveQinQ configurataion does not be implememt on WebUI. |
| ASF4512MP-FLF-EC-00238 | MVR6: MVR6 does not be implemented on WebUI. |
| ASF4512MP-FLF-EC-00187 | IGMP-SNP: After sent IGMP leave message, CLI print debug message. |
| ASF4512MP-FLF-EC-00086 | filename: If it uses too long filename in TFTP image download in Loader environment, command "dir" would work abnormally. |
| ASF4512MP-FLF-EC-00145 | L2SW: Fast pluging SFT module cause error message printed. |
| ASF4512MP-FLF-EC-00314 | System: Shutdown and no shutdown the members of LACP Trunk, CLI shall return error message sometimes. |
| ASF4512MP-FLF-EC-00350 | LAN: LAN_SendMultiPacket cant work correctly |
| ASF4512MP-FLF-EC-00322 | IPv6-ACL: ACE can not be deleted via CLI and WebUI. |
| ASF4512MP-FLF-EC-00349 | need to rebind ACL rules on port, and then New ACL rule can work. |
| ASF4512MP-FLF-EC-00334 | LLDP-MED: The DUT did not support the local information of LLDP-MED objects on standard MIB. |
| ASF4512MP-FLF-EC-00347 | log enhancement : add port information to STA topology change notification |
| ASF4512MP-FLF-EC-00346 | Diag: add fiber port test in diag. external loopback. |
| ASF4512MP-FLF-EC-00332 | SNMP:Can't getbulk from Object Label = amtrMacAddrAgingStatus(Object ID = 1.3.6.1.4.1.1991.1.16.1.1.1.8.3) |
| ASF4512MP-FLF-EC-00282 | Cannot log dying gasp message in oam event log |
| ASF4512MP-FLF-EC-00317 | System: Host loss IGMP group as topology change. |
| ASF4512MP-FLF-EC-00340 | remove output modifier in "show ip arp inspection configuration" |
| ASF4512MP-FLF-EC-00293 | ProtoVlan-MIB: The value of dot1vProtocolGroupRowStatus and dot1vProtocolPortRowStatus are emtpy |
| ASF4512MP-FLF-EC-00284 | OAM: Show interface brief will display wrong link status after unplugging the utp cable. |
| ASF4512MP-FLF-EC-00330 | OAM: [MEF 14CP] DTE must not transmit Remote Stable and Remote Evaluating bits encoded as 0x3. |
| ASF4512MP-FLF-EC-00337 | enhance: multicast change using mac table to ip table |
| ASF4512MP-FLF-EC-00336 | MVR/MVR6 enhance support"proxy-query-interval" and "source-port-mode" command |
| ASF4512MP-FLF-EC-00335 | [LNA-Only] SNMP/LLDP: missing "length" in the variable-length subindex lldpLocManAddr of lldpConfigManAddrTable |
| ASF4512MP-FLF-EC-00339 | show reason if the port is shut down by BPDU guard |
| ASF4512MP-FLF-EC-00338 | IGMSNP:when dynamic/dummy entry cause table full then static registered group port link up, static group will register fail |
| ASF4512MP-FLF-EC-00222 | LLDP: The value of Auto-nego capability in IEEE 802.3 Organizationally Specific TLVs is always 0 |
| ASF4512MP-FLF-EC-00160 | CFM: The SPC should modify it since not support the ConflictingVIDs error. |
| ASF4512MP-FLF-EC-00320 | System: Test overnight, the SNMP-Walk and telnet session always timeout. |
| ASF4512MP-FLF-EC-00325 | System: DUT still generates logs as VLAN link-down, topology change and reset Sys-log even; and send out Trap and Log. |
| ASF4512MP-FLF-EC-00249 | Diffserv: Can't set specific CoS value for traffic class |
| ASF4512MP-FLF-EC-00326 | configuration of synce drop out after switch rebooting. |
| ASF4512MP-FLF-EC-00292 | [Port Security] Behavior of port is not expect after type "no port security" command |
| ASF4512MP-FLF-EC-00258 | MAC: Total MAC count display wrong value. |
| ASF4512MP-FLF-EC-00262 | Port-Trunk: Data flooding with 4port trunk. |
| ASF4512MP-FLF-EC-00331 | IPv6-ACL: Some packets that match permit ACE can not be forwarded when this ACL has maximum ACE. |
| ASF4512MP-FLF-EC-00260 | SNMPv3: DUT sends out cold start trap when doing v1 & v2 warm start. |
| ASF4512MP-FLF-EC-00119 | CLI: The number of loader version not arrange neatly. |
| ASF4512MP-FLF-EC-00324 | System: Test ERPS domains for overnight, the IGMP host loss all multicast groups and SNMP-get timeout.. |
| ASF4512MP-FLF-EC-00311 | LLDP - Show Local Device Info -> System Capabilities fields are blank |
| ASF4512MP-FLF-EC-00312 | LLDP - Show Remote Device Info - System Capabilites fields are blank |
| ASF4512MP-FLF-EC-00327 | IGMPSNP: Configure IGMPSNP priority before host register, the function is not work. |
| ASF4512MP-FLF-EC-00307 | System: Host loss IGMP group as shutdown and no shutdown LACP trunk. |
| ASF4512MP-FLF-EC-00288 | Trunk: Web page display empty info in Trunk-->History page. |
| ASF4512MP-FLF-EC-00131 | System Fault on high CPU, initial ERPS, CFM configuration |
| ASF4512MP-FLF-EC-00310 | missing FTP copy function in web |
| ASF4512MP-FLF-EC-00313 | CFM > Show Information > Show Local MEP Details - AIS Period -> missing unit of measure |

Runtime V1.0.6.3 ; Loader V1.0.0.5

| ASF4512MP-FLF-EC-00342 | ECS4810-12M Link Issue |
| --- | --- |

Runtime V1.0.6.2 ; Loader V1.0.0.5

|  | Port-channel has problem which may block the port. |
| --- | --- |

Runtime V1.0.6.1 ; Loader V1.0.0.5

| ASF4512MP-FLF-EC-00331 | IPv6-ACL: Some packets that match permit ACE can not be forwarded when this ACL has maximum ACE. |
| --- | --- |

Runtime V1.0.6.0 ; Loader V1.0.0.5

| ASF4512MP-FLF-EC-00243 | ome time port is link down, even partner is linkup after cold or warm reboot |
| --- | --- |
| ASF4512MP-FLF-EC-00121 | Reboot DUT while streaming, CLI print debug message and data can't forwarding |
| ASF4512MP-FLF-EC-00276 | SNMP_MAU: ASF4512MP-FLF-EC doesn't support MAU mib. |
| ASF4512MP-FLF-EC-00308 | System: Inject unicast and multicast traffic to DUTs, CLI return error sometimes. |
| ASF4512MP-FLF-EC-00277 | SNMP_rmon: alarm and event function doesn't work fine. |
| ASF4512MP-FLF-EC-00294 | MVR: Multicast date can not be forwarded when join then remove MVR VLAN manually. |
| ASF4512MP-FLF-EC-00245 | System: sometime file system has problem after update and reboot, to cause the system failed. |
| ASF4512MP-FLF-EC-00214 | Reload: Continuous reload (about 100 times) cause DUT hang up. |
| ASF4512MP-FLF-EC-00159 | CFM: The MA creating behavior of CLI not agree with WEB. |
| ASF4512MP-FLF-EC-00321 | System: DUT should not send out Trap-even and log to server for overnight. |
| ASF4512MP-FLF-EC-00110 | OAM: Remote device start oam remote loopback , ECS481012M still showing loop back fail |
| ASF4512MP-FLF-EC-00319 | ISystem: CLI always print Traplog and topology change information. |
| ASF4512MP-FLF-EC-00318 | System: CLI return error continuously as create LACP trunk and 255 VLANs on DUT. |
| ASF4512MP-FLF-EC-00210 | MVR: MVR VLAN on receiver port can be removed manually. |
| ASF4512MP-FLF-EC-00300 | dot1agCfmMepDownTrap description - suggested change |
| ASF4512MP-FLF-EC-00290 | VoiceVLAN: Priority of Output Packet Have Not Be Changed |
| ASF4512MP-FLF-EC-00306 | the HTTPS certificate is not for ES3510MA |
| ASF4512MP-FLF-EC-00305 | lldp protocol block the port of the SMC8126L2 |
| ASF4512MP-FLF-EC-00281 | Power-save :It seems that if I enter the command fully the status is not indicated. |

Runtime V1.0.3.8 ; Loader V1.0.0.5

| ASF4512MP-FLF-EC-00280 | Web: Delete one time-range periodic rule entry, the other rule entries after the delete one won't display. |
| --- | --- |
| ASF4512MP-FLF-EC-00257 | DHCP Snooping: DUT cannot restore the binding entry. |
| ASF4512MP-FLF-EC-00233 | NetworkNotice: syslog is incomplete when interface link up/down |
| ASF4512MP-FLF-EC-00232 | NetworkNotice: Trunk Link Down/Up handling |
| ASF4512MP-FLF-EC-00275 | SNMP_RFC-2618 RadiusAuthClient: radiusAuthClientMalformedAccessResponses |
| ASF4512MP-FLF-EC-00156 | DNS Resolver MIB: Get dsnCacheIntAddress, return invalid value |
| ASF4512MP-FLF-EC-00166 | Private MIB - fileMgt checkting, get invalid value |
| ASF4512MP-FLF-EC-00251 | RSPAN: The mirror function in rspan session 1 doesn't work after creating the 2th rspan session. |
| ASF4512MP-FLF-EC-00247 | LACP: LACP performacne |
| ASF4512MP-FLF-EC-00286 | IPv6-ACL: IPv6 ACL cann't be bound to the port in egress direction after rebooting. |
| ASF4512MP-FLF-EC-00289 | IPv6-ACL: Some packets that match permit ACE can not be forwarded when this ACL has maximum ACE. |
| ASF4512MP-FLF-EC-00202 | IGMP-SNP: Create maximum IGMP filter profile with 508 rang entry, the running configuration can't be saved. |
| ASF4512MP-FLF-EC-00295 | MVR: Can't removt MVR domain type from port-channel |
| ASF4512MP-FLF-EC-00301 | igmp profile loss one profile control table |
| ASF4512MP-FLF-EC-00299 | igmpsnp, static group won't be inherited by trunk port |
| ASF4512MP-FLF-EC-00248 | RSAPN: The port that is enabled port security can not be set as source or destination port. |
| ASF4512MP-FLF-EC-00170 | CLI: Need to add IPv6 address format description in setting DNS&SNTP server. |
| ASF4512MP-FLF-EC-00199 | MVR: Default value of Fwarding Priority and Upstream Source IP is wrong on WebUI. |
| ASF4512MP-FLF-EC-00237 | MVR: Two errors in MVR Configure Domain web page. |
| ASF4512MP-FLF-EC-00201 | MVR: "Expire" and "Count" are wrong when show MVR member from WebUI. |
| ASF4512MP-FLF-EC-00303 | [ES3528M_V1.4.14.1] network-access mode mac-authentication can not work on access port |
| ASF4512MP-FLF-EC-00298 | The Command of TRTCM should be modified |
| ASF4512MP-FLF-EC-00297 | SelectiveQinQ: Behavior is not correct when ingress packets do not match rules on access port. |

Runtime V1.0.3.7 ; Loader V1.0.0.5

| ASF4512MP-FLF-EC-00106 | ECS4810-12M : Wrong word ( Failed to remove "untaged" list on port traunk 1) |
| --- | --- |
| ASF4512MP-FLF-EC-00105 | ECS4810-12M switch will hang when I attempted to perfrom snmp walk on craft port. |
| ASF4512MP-FLF-EC-00117 | CLI: "access-group" can't be showed when we use "show a + ?" command. |
| ASF4512MP-FLF-EC-00128 | If a user disables DHCP on the default vlan interface, it should not inherit the DHCP acquired IP address as a static configu |
| ASF4512MP-FLF-EC-00118 | IGMP_SNP_Filter: Get the MIB object igmpSnoopThrottlePortAction, it always is zero. |
| ASF4512MP-FLF-EC-00147 | Private MIB-vlanMgt: Doesn't support mac vlan private mib. |
| ASF4512MP-FLF-EC-00148 | Private MIB-vlanMgt: Doesn't support subnet VLAN mib. |
| ASF4512MP-FLF-EC-00136 | ACL: Behavior of ACL with ToS filtering is worng. |
| ASF4512MP-FLF-EC-00038 | [ACL] ACL IP precedence doesn't work. |
| ASF4512MP-FLF-EC-00137 | ACE with ToS field doesn't be deleted.. |
| ASF4512MP-FLF-EC-00134 | ACL: ACE that is deny any any doesn't be deleted when ACL has protocol field ACE. |
| ASF4512MP-FLF-EC-00144 | LED: Diagnostic LED is not correct while booting. |
| ASF4512MP-FLF-EC-00146 | RFC-2674 qBridge MIB: The value of dot1qNextFreeLocalVlanIndex is always "0" |
| ASF4512MP-FLF-EC-00132 | ACL: Behavior of ACL with VID filtering is worng. |
| ASF4512MP-FLF-EC-00133 | ACL: Protocol filtering doesn't work |
| ASF4512MP-FLF-EC-00130 | Configuring ERPS RPL Owner via the web interface does not apply it to the ECS |
| ASF4512MP-FLF-EC-00152 | Cluster: Debug message would be showed when the DUT was joined to active cluster member. |
| ASF4512MP-FLF-EC-00154 | Cluster: Cluster active member can not be telnet. |
| ASF4512MP-FLF-EC-00103 | [Broadcast storm]Broadcast speed limit |
| ASF4512MP-FLF-EC-00113 | Storm: The storm rate always is lower than user setting. |
| ASF4512MP-FLF-EC-00173 | PVLAN: After a port join to PVLAN, enable LACP the entry will be changed. |
| ASF4512MP-FLF-EC-00129 | use command shutdown on port channel 1 , the system will lock up |
| ASF4512MP-FLF-EC-00167 | PVLAN: Enter an error command the setting still take effect. |
| ASF4512MP-FLF-EC-00122 | QinQ: DUT can't learn MAC address and DUT would crash. |
| ASF4512MP-FLF-EC-00127 | MACs not learned from hosts forwarding traffic. After reboot, MACs learned |
| ASF4512MP-FLF-EC-00218 | MIB: MgtIPFilter: mgtIPFilter checking |
| ASF4512MP-FLF-EC-00225 | LLDP-MED: Can't enable LLDP-MED notification via CLI and Web. |
| ASF4512MP-FLF-EC-00108 | show mvr6 domain 1 members - syntax error |
| ASF4512MP-FLF-EC-00198 | IGMP-SNP: Can't display IGMP profile on CLI. |
| ASF4512MP-FLF-EC-00204 | IGMP-SNP: Can't configure max-group on port-channel |
| ASF4512MP-FLF-EC-00183 | IGMP-SNP: The host timeout value is incorrect |
| ASF4512MP-FLF-EC-00200 | MVR: The result of sh mvr members IP address is empty via CLI. |
| ASF4512MP-FLF-EC-00206 | MVR: The port does not be removed from MVR VLAN even it became normal port. |
| ASF4512MP-FLF-EC-00231 | MAC-based VLAN: The MAC-vlan still forwarded as remove maximum entries. |
| ASF4512MP-FLF-EC-00250 | SSHv2: SSH Timeout |
| ASF4512MP-FLF-EC-00239 | ProtocolVLAN: Protocol VLAN with type IP is not work properly. |
| ASF4512MP-FLF-EC-00236 | MVR: The trunk interface can not be set to MVR source port. |
| ASF4512MP-FLF-EC-00240 | ProtovolVLAN-MIB: Can't configure protocol VLAN via dot1v protocol MIB |
| ASF4512MP-FLF-EC-00205 | IGMP-SNMP-MIB: Set igmpSnoopCurrentVlanQueryIntvl and igmpSnoopCurrentVlanQueryRespIntvl always reply commitfailed |
| ASF4512MP-FLF-EC-00155 | ERPS: Set ERPS domain to "notReady" via SNMP, the ERPS domain will be deleted. |
| ASF4512MP-FLF-EC-00111 | correction to grammar - show lldp info remote-device |
| ASF4512MP-FLF-EC-00162 | IPv6-AddressConfig: IPv6 MTU can not be set via CLI or WebUI. |
| ASF4512MP-FLF-EC-00163 | IPv6-AddressConfig: IPv6 nd reachable-time can not be set. |
| ASF4512MP-FLF-EC-00267 | ICMP Statistics: ICMP sent destination unreachable messages |
| ASF4512MP-FLF-EC-00263 | PortTrunk: Can change load balance mode via CLI. |
| ASF4512MP-FLF-EC-00234 | Dynamic Provision: DUT fail to get bootfile as use vendor-class ID TO GET IP. |
| ASF4512MP-FLF-EC-00268 | SNMP_TCP: tcpConnTable is no use. |
| ASF4512MP-FLF-EC-00269 | SNMP_TCP: tcpRetransSegs is no use. |
| ASF4512MP-FLF-EC-00278 | SNMP_qBridgeMIB: dot1qNextFreeLocalVlanIndex is no use. |
| ASF4512MP-FLF-EC-00124 | STP-BPDUGuard: The Guard should not enabled as auto-edge enabling. |
| ASF4512MP-FLF-EC-00126 | STP-LoopDetection: The release mode not display in interface. |
| ASF4512MP-FLF-EC-00228 | CFM: MIP not forward some CFM frames with lower level to other ports. |
| ASF4512MP-FLF-EC-00227 | CFM: MIP not forward some CFM frames with same level to other ports. |
| ASF4512MP-FLF-EC-00123 | STP-AutoEdge: Enable auto-edge, the operation status always is disabled. |
| ASF4512MP-FLF-EC-00120 | STP-AutoEdge: Enable auto-edge, the operation status always is disabled. |
| ASF4512MP-FLF-EC-00164 | Diffserv: Configuration disappear after reboot. |
| ASF4512MP-FLF-EC-00186 | IPv6-ACL: ACE with DSCP filtering can not work. |
| ASF4512MP-FLF-EC-00259 | RateLimit: Rate limit on trunk interface can not be configured via WebUI. |
| ASF4512MP-FLF-EC-00188 | IPv6-ACL: Packets that match the second host ACE don't be forwarded. |
| ASF4512MP-FLF-EC-00172 | PVLAN: (Web) Can't delete multiple entry via web |
| ASF4512MP-FLF-EC-00221 | LLDP: Can't enable Port Description/System Capabilities/System Description/System Name TLV on web |
| ASF4512MP-FLF-EC-00224 | LLDP-MED: Please remove MED extended power TLV on web UI. |
| ASF4512MP-FLF-EC-00138 | ACL: Only 909 ACEs can be created. |
| ASF4512MP-FLF-EC-00149 | ACL: time-range, next-header and DSCP configuration in IPv6 ACL would disappear after rebooting. |
| ASF4512MP-FLF-EC-00270 | IEEE-8021PAE MIB: dot1xPaePortInitialize |
| ASF4512MP-FLF-EC-00230 | MAC-based VLAN: Create maximum entries, DUT return error message. |
| ASF4512MP-FLF-EC-00252 | Diffserv: Can't modify calss-map via web if create over 125 class-map. |
| ASF4512MP-FLF-EC-00109 | Web: the web page is incorrect in multicast->IGMP Snooping->Interface |
| ASF4512MP-FLF-EC-00178 | IPSubnet-basedVLAN: The ping session timeout as enable the IPSubnet-based VLAN. |
| ASF4512MP-FLF-EC-00135 | Web: add new web help page |
| ASF4512MP-FLF-EC-00179 | IPSubnet-basedVLAN: CLI should display port configuration information. |
| ASF4512MP-FLF-EC-00150 | MSTP: Get the MIB object xstInstancePortPathCost, it always is zero. |
| ASF4512MP-FLF-EC-00181 | IPSubnet-basedVLAN: Create maximum entries, DUT handled and ping session timeout. |
| ASF4512MP-FLF-EC-00209 | CFM: DUT return the error message as receive stack VLAN CCM. |
| ASF4512MP-FLF-EC-00139 | ACL: It's failed to configure more than on absolute rule in one time range. |
| ASF4512MP-FLF-EC-00182 | IPSubnet-basedVLAN: DUT not reject the invalid address and mask setting. |
| ASF4512MP-FLF-EC-00213 | NetworkMonitor: Bridge-MIB get invalid value |
| ASF4512MP-FLF-EC-00256 | IP Source-guard: DUT not discard the non-security traffic as dynamic entry aging. |
| ASF4512MP-FLF-EC-00212 | RMON MIB: Get probeDateTime, but return invalid value |
| ASF4512MP-FLF-EC-00272 | TrapMgmt: Send LinkDown notification first when the port is up |
| ASF4512MP-FLF-EC-00271 | TrapMgmt: Linkup traps send twice from one port |
| ASF4512MP-FLF-EC-00180 | ATC: Precise value of rate control is not good. |
| ASF4512MP-FLF-EC-00219 | MAC-Mirror: Please add MAC-based mirror MIB. |
| ASF4512MP-FLF-EC-00211 | VLAN-Mirror: Can't read VLAN mirror status via MIB. |
| ASF4512MP-FLF-EC-00151 | MIB: Private MIB compile error |
| ASF4512MP-FLF-EC-00116 | Storm: Get the MIB object unknownUcastStormPktRate, it always is zero. |
| ASF4512MP-FLF-EC-00115 | Storm: Get the MIB object bcastStormPktRate, it always is zero. |
| ASF4512MP-FLF-EC-00114 | Storm: Get the MIB object mcastStormPktRate, it always is zero. |
| ASF4512MP-FLF-EC-00107 | ECS4810-12M CVLAN tag is not being preserved outbound on the CVLAN (access) interface on the ECS4810-12M |
| ASF4512MP-FLF-EC-00104 | ECS4810-12M Q-in-Q doesn't work correctly |
| ASF4512MP-FLF-EC-00125 | STP-LoopDetection: The forwarding status not changed as defect BPDU looping. |
| ASF4512MP-FLF-EC-00203 | CLI: Continuous login fail cause console lock. |
| ASF4512MP-FLF-EC-00177 | ATC: OID of all ATC traps is wrong. |
| ASF4512MP-FLF-EC-00241 | Voice VLAN: DUT should not learn the OUI MAC as change to LLDP rule. |
| ASF4512MP-FLF-EC-00223 | LLDP-MIB: Set lldpPortConfigTLVsTxEnable to 0xF0, but TLVs doesn't enable. |
| ASF4512MP-FLF-EC-00194 | CFM: DUT should not defect the error as global disable the CFM. |
| ASF4512MP-FLF-EC-00196 | CFM: DUT should not send out the fault-alarm Trap as global disable the CFM. |
| ASF4512MP-FLF-EC-00161 | IPv6-AddressConfig: Disable IPv6 explicitly from WebUI would pop "Data is invalid" window. |
| ASF4512MP-FLF-EC-00184 | IPv6-ACL: There are no source IPv6 address items to set via WebUI. |
| ASF4512MP-FLF-EC-00215 | MIB: MgIPFilter-Management IP filter for HTTP client |
| ASF4512MP-FLF-EC-00216 | MIB: MgIPFilter-Management IP filter for Telnet client |
| ASF4512MP-FLF-EC-00217 | MIB: MgIPFilter-Management IP Filter for SNMP Client |
| ASF4512MP-FLF-EC-00273 | STDMIB: ifOperStatus shows 7: lowerLayerDown when port is down(shutdown or no link) |
| ASF4512MP-FLF-EC-00264 | SNMP: The function of ifInDiscards doesn't work. |
| ASF4512MP-FLF-EC-00140 | QoS: Traffic with high drop precedence but not drop first. |
| ASF4512MP-FLF-EC-00157 | DNS: Type ESC to abort DNS resolve operation failed |
| ASF4512MP-FLF-EC-00253 | PortMirror: Packets lost would occur when one high speed port monitor multiple low speed ports. |
| ASF4512MP-FLF-EC-00030 | SFP can not linkup |
| ASF4512MP-FLF-EC-00169 | DelayReload, the system can't detect specific time very well |
| ASF4512MP-FLF-EC-00171 | DealyReload, the system can't detect the correct time settings |
| ASF4512MP-FLF-EC-00185 | IPv6-ACL: Range of destination IPv6 address prefix-length shall be pointed out. |
| ASF4512MP-FLF-EC-00158 | CFM: The domain index range is incorrect. |
| ASF4512MP-FLF-EC-00142 | ACL: Rules in one time-range can exceed 8 entries via WebUI. |
| ASF4512MP-FLF-EC-00165 | SNMP-MIB: The CFM trap not define in MIB object "ecs4810-12mTrapsPrefix". |
| ASF4512MP-FLF-EC-00175 | CFM: DUT not defect the Xcon as receive lower level CCM. |
| ASF4512MP-FLF-EC-00229 | CFM: Use new standard CFM MIB not get any value from DUT. |
| ASF4512MP-FLF-EC-00244 | CFM: DUT lose the CFM trunk configuration after save running configuration |
| ASF4512MP-FLF-EC-00176 | CFM: DUT not defect the remoted miss as not receive all. |
| ASF4512MP-FLF-EC-00190 | CFM: Receive RDI CCM, DUT not defect the RDI error. |
| ASF4512MP-FLF-EC-00191 | CFM: When the MACStatus defected and receive one interface TLV with Up, DUT not clear the error. |
| ASF4512MP-FLF-EC-00192 | CFM: The MACStatus should not defected unless all remoted CCM with Interface down. |
| ASF4512MP-FLF-EC-00193 | CFM: When DUT defect the remoted and receive RDI CCM from same MEP, the remoted defection not change to RDI. |
| ASF4512MP-FLF-EC-00197 | CFM: When DUT defect the MACStatus and receive same CCM with RDI and port Up, DUT not change to RDI defection. |
| ASF4512MP-FLF-EC-00207 | CFM: DUT not send out the MA-UP Trap as receive all MEPs with MAC-status Up. |
| ASF4512MP-FLF-EC-00261 | Banner: Can't display login banner. |
| ASF4512MP-FLF-EC-00242 | IPv6-AddressConfig: DHCPv6 solict can not be sent when enabled/disabled IPv6 address autoconfig repeatedly. |
| ASF4512MP-FLF-EC-00235 | IPv6-AddressConfig: The system only accepts 15 prefix option in one RA. |
| ASF4512MP-FLF-EC-00143 | IP fails to reach beyond default gateway after moving IP address from craft interface to default vlan |

Runtime V1.0.3.4 ; Loader V1.0.0.4

| ASF4512MP-FLF-EC-00101 | [Trunk] join port to a trunk will display error message |
| --- | --- |
| ASF4512MP-FLF-EC-00100 | Enhancement: add sflow function |

Runtime V1.0.3.3 ; Loader V1.0.0.4

| ASF4512MP-FLF-EC-00068 | WebUI: add counter(VLAN, history), Trunk algorithm selection |
| --- | --- |
| ASF4512MP-FLF-EC-00081 | WebUI: add transceiver(include DDM) |
| ASF4512MP-FLF-EC-00083 | corrections to grammar on web |
| ASF4512MP-FLF-EC-00069 | QoS: bind one meter on a port but it will take effect for system |
| ASF4512MP-FLF-EC-00080 | QoS: get-walk from diffservPortTable will fail |
| ASF4512MP-FLF-EC-00061 | QoS: Fail to set "qos map dscp-mutation" |
| ASF4512MP-FLF-EC-00071 | shared memory overlay stack memory, cause some functions crash |
| ASF4512MP-FLF-EC-00043 | add CLI command to setup algorithm for trunk by Poncat capability |
| ASF4512MP-FLF-EC-00070 | ERPS: traffic recover too slow when port link up at 1000M |
| ASF4512MP-FLF-EC-00072 | DIAG: shall not link core layer lib |
| ASF4512MP-FLF-EC-00073 | spelling correction - show mvr domain member |
| ASF4512MP-FLF-EC-00089 | [Loader] Need to modify loader so that Trace32 can attach |
| ASF4512MP-FLF-EC-00090 | [enhance] STP: add block & shutdown functions in STP |
| ASF4512MP-FLF-EC-00085 | IEEE: adjust register in 100 Mbps & 10 Mbps for VOD and harmonic |
| ASF4512MP-FLF-EC-00091 | mvr/mvr6 profile support multiple range |
| ASF4512MP-FLF-EC-00095 | MVR support pxoy-switching |
| ASF4512MP-FLF-EC-00096 | EPRs linkdown forwading port won't notify igv3snp cause igmpsnp mvr vlan interface down |
| ASF4512MP-FLF-EC-00094 | Web: update new web help files |
| ASF4512MP-FLF-EC-00046 | Change L2 MAC table size from 8K to 16K |
| ASF4512MP-FLF-EC-00087 | OAM: Timing issue for entering remote loopback mode with testcenter. |
| ASF4512MP-FLF-EC-00012 | WebUI add G.8032, OAM & CFM function and web pages |
| ASF4512MP-FLF-EC-00007 | Modify stktplg for ASF4512MP R0A board |
| ASF4512MP-FLF-EC-00021 | There have error messages in system startup |
| ASF4512MP-FLF-EC-00098 | Cable diagnostics problem of Pair A & Pair B |
| ASF4512MP-FLF-EC-00093 | CPU Tx/Rx packets via SDMA mode instead of MII mode |
| ASF4512MP-FLF-EC-00044 | port status is sometimes incorrect: MAC is down and PHY is up. |
| ASF4512MP-FLF-EC-00017 | Add 15 minutes and 1 day counter function |
| ASF4512MP-FLF-EC-00011 | Add counter per VLAN function |
| ASF4512MP-FLF-EC-00084 | loader: smi command work abnormal |
| ASF4512MP-FLF-EC-00016 | 802.3ah support both CO and CPE modes |
| ASF4512MP-FLF-EC-00041 | (CFM)To support unique ma name per domain/frame delay variation. |
| ASF4512MP-FLF-EC-00088 | CFM: Use CPU MAC for up MEP. |
| ASF4512MP-FLF-EC-00010 | Detect power AC <-> DC switching and send trap |
| ASF4512MP-FLF-EC-00014 | DDM can setup threshold and send trap, and can setup in MIB |
| ASF4512MP-FLF-EC-00024 | Add alarm function |
| ASF4512MP-FLF-EC-00092 | Add diagnostic functions |
| ASF4512MP-FLF-EC-00099 | DDM: when alarm & warning status is gone and back to normal, DDM should send trap to notice |

Runtime V1.0.2.2 ; Loader V1.0.0.1

| ASF4512MP-FLF-EC-00060 | GVRP: Let GVRP PDU flood when GVRP is disable |
| --- | --- |
| ASF4512MP-FLF-EC-00064 | enhance: subnet vlan per port bind |
| ASF4512MP-FLF-EC-00065 | Enhancement: VLAN behavior consistency |
| ASF4512MP-FLF-EC-00062 | WebUI: web show queue weight is incorrect for value 255 |
| ASF4512MP-FLF-EC-00063 | WebUI: add new QoS function to web |
| ASF4512MP-FLF-EC-00013 | WebUI modify MVR function and web pages |
| ASF4512MP-FLF-EC-00067 | SNMP: Trap packets could not been sent via craft port (management port) |
| ASF4512MP-FLF-EC-00066 | AutoNeg: cannot enable autoneg on trunk from CLI |
| ASF4512MP-FLF-EC-00042 | add one rule to remap IEEE 1588 packet to highest queue when CLI command enable |
| ASF4512MP-FLF-EC-00059 | ClassMap: CLI show failed if input match cos 0 after match vlan 1001 in a class-map |
| ASF4512MP-FLF-EC-00057 | Enhance: enlarge chip queue size |
| ASF4512MP-FLF-EC-00058 | QueueMode; queue mode will be wrong after config queue weight |
| ASF4512MP-FLF-EC-00047 | mvr associated-profile & mvr domain associate-profile |
| ASF4512MP-FLF-EC-00056 | QOS: Check rule resource for qos is incorrect |
| ASF4512MP-FLF-EC-00055 | Trunk : "Trunk Load Balance Mode: Source MAC address" doesn't work. |
| ASF4512MP-FLF-EC-00051 | Save running configure has problem for qos map phb-queue |
| ASF4512MP-FLF-EC-00052 | EgressRateLimit: output rate of two ports might be different when they have identical egress rate limit config |
| ASF4512MP-FLF-EC-00039 | (Duplicate ASF4626BA-FLF-VE-00013)[ACL] If a ACL binding MAC conflicts with another ACL binding IP, DUT shall obey the ACL b |
| ASF4512MP-FLF-EC-00053 | ClassMap: no match cos and no match source-port is not working |
| ASF4512MP-FLF-EC-00050 | ACL: port add ip standard rule, but it can not show up |
| ASF4512MP-FLF-EC-00036 | [Port Mirror] If it uses one port to monitor both receiving of two ports, the monitoring result should be about 50% for each |
| ASF4512MP-FLF-EC-00049 | mvr group entry and igmp snp dynamic entry in same group exist at same time |
| ASF4512MP-FLF-EC-00018 | Modify function for bundle rate limit |
| ASF4512MP-FLF-EC-00040 | [ProtocolVlan] protocol-vlan vlan priority shall be from 0 to 7 |
| ASF4512MP-FLF-EC-00023 | ACL configuration have error messages |
| ASF4512MP-FLF-EC-00037 | [ACL] DUT shows debug message when a ACL rule binds to a port. |
| ASF4512MP-FLF-EC-00045 | igmpsnp and mvr statistics without output counter |
| ASF4512MP-FLF-EC-00034 | [Port Security] The port shall be able to learn the MAC addresses as Port Security MAC until the Max-Mac-Count is reached. |
| ASF4512MP-FLF-EC-00026 | [Protocol Vlan] Set protocol protocol on port, it causes DUT exception. |
| ASF4512MP-FLF-EC-00035 | [Port Security] If Max-Mac-Count configures to zero, it shall make Port Security function to be disabled. |
| ASF4512MP-FLF-EC-00025 | [IGMPSNP] If it changes Vlan state to "suspend" and "no ip igmp snooping", DUT will be hang-up. |
| ASF4512MP-FLF-EC-00033 | (ST-0085)[Cfg Mgmt] - Copy and paste the message of .cfg file's content to CLI, the CLI of serial had been freezed. |
| ASF4512MP-FLF-EC-00005 | WRR queue weight can not be any order |
| ASF4512MP-FLF-EC-00015 | Modify ingress rate limit to 2 stage for PonCat |
| ASF4512MP-FLF-EC-00020 | Cos setting has error message |
| ASF4512MP-FLF-EC-00019 | trunk port can not do MAC learning |
| ASF4512MP-FLF-EC-00006 | Update uboot code for ASF4512MP |
| ASF4512MP-FLF-EC-00028 | ARP cannot be switched. |
| ASF4512MP-FLF-EC-00003 | Implement MVR new features |
| ASF4512MP-FLF-EC-00022 | strict+WRR setting cause system crash. |
| ASF4512MP-FLF-EC-00031 | [Flow Control] DUT sends Pause frames out, even if traffic loading isn't overload. |
| ASF4512MP-FLF-EC-00008 | Add dying gasp function |
| ASF4512MP-FLF-EC-00054 | Add Sync-E function |

Known Issue

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

- Vlan trunking can be supported on all ports. The manual is incorrect on “VLAN trunking can only be enabled on Gigabit Ethernet port or trunks”

- VLAN mirroring or MAC-based mirroring can not coexist with Port mirroring. While VLAN mirroring and MAC-based mirroring can be coexisted, but they must use the same target port.

- MAC Chip limit: unicast, multicast, broadcast and storm control all use one same threshold, although each one can enable/disable independent. If user need to use different threshold for each one, he must use rule (diffserv) to do it.

- PHY chip limit: Cable Test on Power Saving is highly inaccurate. If you want to get accurate cable test, please turn off power saving mode first.

- PHY chip limit: GE port can do cable diagnostic in linkup, but FE & GE ports can only do cable diagnostic in linkdown without partner

- PHY chip limit: Power saving (green Ethernet) is work for GE port only.

- MAC chip limit: There is no HW loopback detection without STP (loop-free detection)

- MAC chip limit: Can not support multiple uplink ports on one session

- No mac-learning command for CLI and webUI, the manual is incorrect.

- As per MVL FAE advice - AlleyCat2/PonCat2 chip limitation... When CCFC (cross-chip-flow-control) is enabled, CCFC doesn’t support to pause ingress stream of trunk port