Access Switch ASX3528

Firmware Version 1.3.12.1

INTRODUCTION:

The ASX3528 is a cost effective managed Fast Ethernet Stackable Layer 2 access Switch. Each ASX3528 unit contains 24 10/100 /100 Based-X/100 Based-T ports with two expansion slots and two combo ports. The slots can support expansion modules such as 1000Base-T, 1000Base-SX, 1000Base-LX and 100FX modules.

The base hardware is a 1.5U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator (shall be Version 6.2 or above) or Microsoft’s Internet Explorer (should be Version 5.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ASX3528 from a RJ45 type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out of band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release note prior to the installation or upgrade of this product.

Firmware Specification:

Runtime

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| Patch release | 1.3.12.1 | Customer | 1/9/2013 |

Daig

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| NTC release | 1.0.1.0 | NTC | 10/12/2012 |

Loader

| Status | Version No. | # Type | # Release Date |
| --- | --- | --- | --- |
| NTC release | 1.0.1.1 | NTC | 11/05/2008 |

LOADER CODE modifications:

###### 1.0.1.1

1. boot up unsuccessfully after power off 8 hours(RFI with invalid branch)

DIAGNOSTIC CODE modifications:

###### 1.0.1.0

the 2nd source flash cause the system excpetion during the switch boot

Runtime  CODE modifications:

###### 1.3.12.1

1. Can't set the parameter of rate-limit input as console

2. snmpwalk (ifMauIfIndex 1.3.6.1.2.1.26.2.1.1.1) may cause switch hangs up

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

Installation and Configuration Notes:

In general, the ASX3528 Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ASX3528 Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

###### 1.3.10.0

1. Support Show interface brief

2. Support ip dhcp snooping information option remote-id ip-address

3. Support Password length up to 10

###### 1.3.8.0

1. Support LLDP

2. DHCPSNP binding entry from 8 to 10

3. Enhancement: CLI show more than one page, need to provide guideline

###### 1.3.6.0

1. Support Bootp snooping

2. Support PSEC MAC filter

###### 1.3.2.0 & 1.3.4.0

1. Support DDM

2. Support OAM remote port status link status/speed duplex

3. enlarge MVR group from 255 to 1024 but active group number is 255 still due to chip limitation

###### 1.3.0.1

1. Support NTP

2. Support HTTP download/upgrade

3. support WEB/MIB of 1.2.1.0/1.1.2.1 new features

###### 1.3.0.0

1. Support EAPS

1. Support IP CLUSTER

###### 1.2.1.0

1. Support L2 tunnel STA/CDP/VTP

2. Support Y.1731

3. Support Tacacs accouting

4. Support Auto upgrade

5. Support FTP support

6. Support Dynamic provision

7. Support Show CPU process

###### 1.1.2.1

1. Support Dynamic ARP inspection

2. Support Dynamic Vlan trunking

3. Support CDP/PVST filter per port

4. Support Port auto edge/loop detection/BPDU flooding

###### 1.0.4.0

1. Support 802.3ah OAM function: Remote firmware upgrade, Standard MIB, Private MIB (remote loopback test, remote firmware upgrade, remote control)

###### 1.0.3.0

1. Support 4094 VLAN.

2. New Web feature: VLAN-based Priority

3. New Web feature: Input CoS Rate Limit Port/Trunk Configuration

4. New Web feature: BPDU Flooding (STA Port/Trunk Configuration)

5. New Web feature: Auto Traffic Control (Timer/Port Configuration, Control Release)

6. New Web feature: MDIX Port Configuration

7. New Web feature: Port Trap Managers (SNMP Configuration)

8. New Web feature: Alarm Information

9. New Web feature: AAA (Accounting Settings, Radius Group Settings, Update Periodic, 802.1x Port Settings, Exec Settings, Summary)

10. New Web feature: QinQ (System Mode Configuration, VLAN Port/Trunk Configuration)

11. New Web feature: Egress Block (Port/Trunk Egress Packet Block)

12. New Web feature: Flash Log

13. New Web feature: Port/Trunk Multicast Control

14. New Web feature: Port/Trunk Unknown Unicast Control

15. New Web feature: Port/Trunk Edge Port Configuration

16. New Web feature: STA Root Guard (STA Port/Trunk Configuration)

17. support BPDU Guard

18. support Root Guard

19. support BPDU filtering

20. support IGMP filtering

21. support IGMP throttling

22. support RMON II(probeConfig, 19)

###### 1.0.2.0

1. When spanning tree is disable BPDU forwarding enable/disable per port

2. Storm Auto traffic control (ATC)

3. IPv6 traffic class remapping

4. MDIX - Auto/MDI/MDIX

5. Link up/link down trap per port configuration

6. SP+WRR

7. Rate limit resolution from 1M to 64K

8. username:adminand prevlige should be read only

9. Need to display Ethertype in interface switchport

10. Added the Alarm(show alarm via CLI)

###### V1.0.0.9  design change

1. module type checking

Known Restrictions and Limitations:

1.0.2.1

1. There is a limitation when we fix IP port cos for ASE3528B-E1-FLF-TER-00086. packet to CPU packet will not allow to configure like DHCP 67/68. 2. There is a limitation on craft port design which may have control packet pass through this like IGMP querier and MVR and add craft port into the group.

1.0.0.9

1. Diffserv:after change the IP DSCP of the DSCP traffic class,the new DSCP can't shall reflect the receiving ratio. Current design need to configure MAP IP DSCP first then configure POLICY-MAP

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2006 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ac | 1000Base-Tx |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| IEEE 802.1X | IEEE8021-PAE-MIB | - dot1xPaeSystem group - dot1xPaeAuthenticator group - dot1xPaeSupplicant group |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | coldStart warmStart linkDown linkUp authenticationFailure |
| RFC 1493 | newRoot topologyChange |
| RFC 2819 | risingAlarm fallingAlarm |