Fast Ethernet Switch ES3528M-SFP-38

Firmware Version 1.4.20.9

INTRODUCTION:

The ES3528M-SFP is a 24-port 100-FX L2 standalone management+ 2 Combo + 2 Gigabit Ethernet Switch. The switch configuration is 24 100-FX mode fiber ports, 2 combo ports and 2 10/100/1000 Base-T ports, which are comprised of an RJ45 and a SFP.

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RJ-45 RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common HTTP browsers such as Netscape Navigator (Version 6.2 or above) or Microsoft’s Internet Explorer (Version 5.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the ES3528M-SFP from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

<Note>

DDM is only supported on combo ports (25-28). Front ports are not supported.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

Runtime

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.4.20.9 | Release | 2012/11/22 |

Diag

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.3.0.0 | Release | 20010/4/14 |

Loader

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Current Version | 1.0.4.0 | Release | 2010/3/9 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Layer 2 Features |  |  |  |
| --- | --- | --- | --- |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 8 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE | YES 1) Total 8 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE |
|  | Cisco EtherChannel Liked | YES 1) Total 8 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE | YES 1) Total 8 Trunks (including Cisco EtherChannel Like trunks 2) 2~8 port/trunk FE 3) 4 port/trunk GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES 1) Unicast : MAC SA/DA 2) Multicast : ???? | YES 1) Unicast : MAC SA/DA 2) Multicast : ???? |
| VLAN | IEEE 802.1Q Tagged Based | YES | YES |
|  | Port-Based | YES (255 VLANs) | YES (4K VLANs) |
|  | Private VLAN | YES (Community) isolated PVLAN | YES (Community) isolated PVLAN |
|  | GVRP | YES | YES |
|  | 802.1v Protocol | YES(global setting) | YES(global setting) |
|  | voice VLAN | YES | YES |
|  | MAC-based VLAN | YES | YES |
|  | IP-subnet VLAN | YES | YES |
|  | VLAN trunking | YES | YES |
| Spanning Tree | IEEE 802.1D | YES | YES |
|  | IEEE 802.1s | YES (8 instances) | YES (8 instances) |
|  | IEEE 802.1w | YES | YES |
|  | Spanning Tree Fast Forwarding | YES | YES |
|  | loop back detection | YES | YES |
|  | auto edge port | YES | YES |
|  | BPDU filter/guard | YES | YES |
|  | root guard | YES | YES |
| Loopback detection (non-STP) |  | NO | NO |
| Broadcast, Multicast, UnknownUnicast Strom Control |  | YES ( bit/sec ) 1) per port rate control 2) range: 64K bits/sec - 100M bits/sec 3) resolution: 64K bits/sec 4) Broadcast, B+M, B+M+DLF | YES ( bit/sec ) 1) per port rate control 2) range: 64K bits/sec - 100M bits/sec 3) resolution: 64K bits/sec 4) Broadcast, B+M, B+M+DLF |
| IGMP Snooping | IGMP Snooping v1/v2 | YES (256 Groups for Static/dynamic, 32 router port) | YES (256 Groups for Static/dynamic, 32 router port) |
|  | IGMP v1/v2 querier support | YES | YES |
|  | IGMP Immediate Leave | YES | YES |
|  | IGMP Filtering/Throttling | YES | YES |
|  | IGMPSNP Leave Proxy | YES | YES |
|  | IGMPV3 Snooping | YES | YES |
| MVR |  | YES | YES |
| Jumbo Frame |  | 10K (only supported in Giga) | 10K (only supported in Giga) |
| QinQ |  | YES | YES |
| PPPoE Intermediate Agent |  | YES | YES |
| EAPS |  | YES | YES |
| BPDU Tunneling |  | YES | YES |
| G.8032 |  | YES | YES |
| QoS Features |  |  |  |
| Number of pripority queue |  | 4 queues/port | 4 queues/port |
| Scheduling for pripority queue | WRR Pripority scheduling | YES Fixed WRR weight 1:2:4:8(no way to change) | YES Fixed WRR weight 1:2:4:8(no way to change) |
|  | Strict Pripority scheduling | YES Once Strict Priority is selected, WRR can not work | YES Once Strict Priority is selected, WRR can not work |
|  | Hybrid (WRR + Strict) Pripority scheduling | NO | NO |
| COS | 802.1p based COS | YES (per system) | YES (per system) |
|  | IP TOS Precedence based COS | NO | NO |
|  | IP DSCP based COS | DSCP Only (per system) | DSCP Only (per system) |
|  | TCP/UDP Port based COS | NO (Doable but need to sacrifise ACL rules) | NO (Doable but need to sacrifise ACL rules) |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) FE: - 64K bits/sec ~ 100Mbits/sec - Resolution 64K bits/sec 3) GE: - 64K bits/sec ~ 1000Mbits/sec - Resolution 64K bits/sec | YES 1) Per Port-based 2) FE: - 64K bits/sec ~ 100Mbits/sec - Resolution 64K bits/sec 3) GE: - 64K bits/sec ~ 1000Mbits/sec - Resolution 64K bits/sec |
|  | Egress | YES 1) Per Port-based 2) FE: - 64K bits/sec ~ 100Mbits/sec - Resolution 64K bits/sec 3) GE: - 64K bits/sec ~ 1000Mbits/sec - Resolution 64K bits/sec | YES 1) Per Port-based 2) FE: - 64K bits/sec ~ 100Mbits/sec - Resolution 64K bits/sec 3) GE: - 64K bits/sec ~ 1000Mbits/sec - Resolution 64K bits/sec |
| DiffServ |  | YES | YES |
| Security Features |  |  |  |
| Static/Dynamic Port Security (MAC-based) |  | YES 1) Per system: Total 1024 MAC 2) Auto-learned support | YES 1) Per system: Total 1024 MAC 2) Auto-learned support |
| 802.1x | Port-Based | YES 1) Support Single host mode 2) Single host mode: - Per port 1 MAC 3) MD5, PEAP,TLS, TTLS | YES 1) Support Single host mode 2) Single host mode: - Per port 1 MAC 3) MD5, PEAP,TLS, TTLS |
|  | Mac-Based | YES | YES |
|  | VLAN assignment | YES | YES |
|  | QoS assignment | YES | YES |
|  | guest VLAN | YES | YES |
|  | Supplicant Support | NO 1) support Win XP 2) support Win 2000 w/ SP4 3) support Win 95/98 w/ AEGIS | NO 1) support Win XP 2) support Win 2000 w/ SP4 3) support Win 95/98 w/ AEGIS |
|  | Auto security | YES | YES |
|  | EAPOL frames pass-through | YES | YES |
| ACL | Number of rules | Mixed mode: 1024 rules Extended mode: 500 rules (per system) | Mixed mode: 1024 rules Extended mode: 500 rules (per system) |
|  | L2/L3/L4 | YES 1) L2 SA/DA MAC, VLAN based 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4)IPv6 | YES 1) L2 SA/DA MAC, VLAN based 2) L3 IP SA/DA, subnet based 3) L4 TCP/UDP port 4)IPv6 |
|  | Time-based | YES | YES |
| RADIUS authentication |  | YES 1) Encryption : MD5 2) Multiple RADIUS Server: 5 servers | YES 1) Encryption : MD5 2) Multiple RADIUS Server: 5 servers |
| RADIUS authentication and accounting |  | YES | YES |
| TACACS+ Authentication |  | YES 1) Encryption : MD5 2) Single Server | YES 1) Encryption : MD5 2) Single Server |
| TACACS+ authorizaion and accounting |  | YES | YES |
| TACACS+ 3.0 ? |  | YES | YES |
| MAC Based Authentication |  | YES | YES |
| HTTPS and SSL (Secured Web) |  | YES Support cipher: 1) AES128-cbc 2) 3ES-cbc 3) Blowfish-cbc | YES Support cipher: 1) AES128-cbc 2) 3ES-cbc 3) Blowfish-cbc |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES Support cipher: 1) AES128-cbc 2) 3ES-cbc 3) Blowfish-cbc | YES Support cipher: 1) AES128-cbc 2) 3ES-cbc 3) Blowfish-cbc |
| User name password authentication | Local Authentication | YES Cisco-like (5 User/password) | YES Cisco-like (5 User/password) |
|  | Remote Authentication via RADIUS/TACACS+ | YES Cisco-like | YES Cisco-like |
| Management Interface Access Filtering | SNMP | YES | YES |
|  | WEB | YES | YES |
|  | TELNET | YES | YES |
| Intrusion lock (link detection) |  | YES | YES |
| MAC filter |  | YES | YES |
| MAC Notification |  | YES | YES |
| Management Features |  |  |  |
| Cisco Like CLI |  | YES | YES |
| CLI filtering and terminal setting |  | YES | YES |
| Web Based Management |  | YES | YES |
| Telnet | Server | YES (8 Sessions) | YES (8 Sessions) |
|  | Client | NO | NO |
| Software Download/Upgrade | TFTP | YES (Boot code, loader, diag) | YES (Boot code, loader, diag) |
|  | Xmodem | YES (Boot code, loader, diag) | YES (Boot code, loader, diag) |
|  | Diag support TFTP download | YES | YES |
|  | FTP | YES | YES |
|  | HTTP | YES | YES |
| Dual Image |  | YES | YES |
| Configuration Download/Upload | TFTP | YES (multiple copies) | YES (multiple copies) |
| Auto Upgrade | TFTP | YES | YES |
| SNMP | v1 | YES 1) 5 SNMP community string 2) 5 trap receivers | YES 1) 5 SNMP community string 2) 5 trap receivers |
|  | v2c | YES 1) 5 SNMP community string 2) 5 trap receivers | YES 1) 5 SNMP community string 2) 5 trap receivers |
|  | v3 | YES | YES |
| RMON | RMON I (1,2,3,9 group) | YES 1) Statistic: - Total 24 entries - Per port 1 entry 2) Histroy: - Total 48 entries - Per port 2 entries 3) Alarm : 48 entries 4) Event : 24 entries (Note: RMON configure will not be saved to Flash) | YES 1) Statistic: - Total 24 entries - Per port 1 entry 2) Histroy: - Total 48 entries - Per port 2 entries 3) Alarm : 48 entries 4) Event : 24 entries (Note: RMON configure will not be saved to Flash) |
| BOOTP | Client | YES | YES |
|  | Relay | NO | NO |
| DHCP | Client | YES | YES |
|  | Relay | NO | NO |
|  | Snooping | YES | YES |
|  | Snooping Option 82 | YES | YES |
|  | Server | NO | NO |
|  | dynamic provision (option 66,67) | YES | YES |
|  | Relay Option 82 | YES | YES |
| IP Source Guard |  | YES | YES |
| Port Mirroring |  | YES (1Dst to manySrc mirroring) | YES (1Dst to manySrc mirroring) |
| Event/Error Log | Local Flash | YES 1) 512 entries 2) 256Kbyte Flash | YES 1) 512 entries 2) 256Kbyte Flash |
|  | Remote server via System Log (RFC3164) | YES 1) Cisco-like 2) Support up to 5 Servers | YES 1) Cisco-like 2) Support up to 5 Servers |
|  | SMTP | YES | YES |
| DNS | Client | YES | YES |
|  | Proxy | YES | YES |
| Remote Ping ? |  | YES | YES |
| SNTP |  | YES 1) Support 3 Servers 2) User configurable | YES 1) Support 3 Servers 2) User configurable |
| NTP |  | YES | YES |
| IP Clustering |  | Yes(36 members) | Yes(36 members) |
| LLDP(802.1ab, Link Layer Discovery Protocol) |  | YES | YES |
| UPnP |  | YES | YES |
| Banner |  | YES | YES |
| WEB auth |  | YES | YES |
| OAM |  | YES | YES |
| sFlow |  | YES | YES |
| Dynamic ARP inspection (DAI) |  | YES | YES |
| VLAN mirror |  | YES | YES |
| MAC Based mirror |  | YES | YES |
| ATC |  | YES | YES |
| Delay reload |  | YES | YES |
| Cable Diagnostics /TDR |  | YES | YES |
| System resources - CPU util and Memory status |  | YES | YES |
| Port Utilization |  | YES | YES |

Installation and Configuration Notes:

In general, the ES3528M-SFP Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES3528M-SFP Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Runtime V 1.4.20.9

| ES3528M-SFP-FLF-38-00262 | dot1qtunnel access mode will not stripped off outer tag on port 1-24 |
| --- | --- |

Runtime V 1.4.20.8

| ES3528MO 1-FLF-38-00073 | inject "OSPF Hello" to ES3528M, the port received this packet will become mrouter port. |
| --- | --- |
| ES3528MO-FLF-38-01147 | ip dhcp snooping trust disappear from LACP port channel after switch rebooting |
| ES3528MO-FLF-38-01145 | If i set wrong value to diffServActionList by snmp, device will hang(console and any management traffic) |
| ES3528M-SFP-FLF-38-00256 | [ES3528M-SFP] there is no any output message when showing interface transceiver |

Runtime V 1.4.20.7 same bug fix as V 1.4.20.6

Runtime V 1.4.20.6

| ES3528MO-PoE-FLF-38-00027 | When we use command "copy tftp running", we will face exception. |
| --- | --- |
| ES3528MO-FLF-17-00075 | switch will hang when inject lldp packets |
| ES3528MO-FLF-38-01146 | Allocate memory size xxx fail memory fail |
| ES3528MO-FLF-38-01054 | LLDP: Memory leaks occur when receiving a PDU which has many Port and Proto VLAN ID TLV of VLAN id |
| ES3528MF-PoE-FLF-17-00003 | SMC6128PL2-f web planel display incorrectly. |

Runtime V 1.4.20.5

| ES3528MO-FLF-38-01141 | Mac-vlan doesn't work with “map ip dscp”and DHCP Snooping |
| --- | --- |
| ES3528MO 1-FLF-38-00070 | flooding report will cause PC start to run report suppresion and won't reply g-s query |

Runtime V 1.4.20.3

| ES3528MO-FLF-38-01144 | ES3528M/ES3552M reboot after SNMP exception when access this node dot3adTablesLastChanged |
| --- | --- |
| ES3528MO-FLF-38-01143 | Support multiple Network Policy TLVs in a LLDP packet |
| ES3528MO-FLF-38-01142 | SDK MAC learning memory leakage problem |
| ES3528MO-FLF-38-01140 | memory leakage issues in CLI and LLDP |
| ES3528MO-FLF-38-01139 | The callback of VLAN status change process may miss some VLANs |
| ES3528MO-FLF-38-01134 | Add addtion checking when parsing LLDP TLV |
| ES3528MO-FLF-38-01133 | Backdoor through telnet will cause memory leakage |

Runtime V 1.4.20.1 Diag V 1.2.1.3; Loader V1.0.4.1

| ES3528MO-FLF-38-01132 | ES3528M - sh spa + press space = lose mngmt (Telnet) |
| --- | --- |
| ES3528MO-FLF-38-01131 | add the last reporter of ip address for IGMP |
| ES3528MO-FLF-38-01129 | [ES3528M_V1.4.18.2] Lost MAC entry in mac-address-table |
| ES3528MO-FLF-38-01128 | ES3528M drop igmp v3 report which has group record 224.0.0.x |
| ES3528MO-FLF-38-01127 | [ES3528M_V1.4.18.2] Should not be used disable DHCPSNP vlan x tagged/untagged function. |
| ES3528MO-FLF-38-01126 | Impossible to backup running-config to TFTP server |
| ES3528MO-FLF-38-01124 | Output filters do not work from “sho int brief” |
| ES3528MO-FLF-38-01120 | STP root change notice enhancement |
| ES3528MO-FLF-38-01115 | Exception messages will appear if ES3528M telnet to another one, show running. |
| ES3528MO-PoE-FLF-38-00026 | LEDDRV_Init fail |
| ES3528MO-PoE-FLF-38-00025 | Bootp can't binding in the dhcp snooping binding table |
| ES3528MO-PoE-FLF-38-00024 | "no power inline compatible" does not take effect |
| ES3528MO-FLF-38-01118 | Auto-Traffic-Control is taking too much CPU usage |

Runtime V 1.4.18.1 Diag V 1.2.1.3; Loader V1.0.4.1

| ES3528MO-FLF-38-00873 | There is nothing about "add-to-running-config" by snmp in the MIB file of ES3528M |
| --- | --- |
| ES3528MO-FLF-38-01092 | [ES3528M_V1.4.16.1] Switch is hanging when user issue clear ip igmp snooping groups dynamic |
| ES3528MO-FLF-38-01094 | [ES3528M_V1.4.16.1] No ip DHCP snooping verify mac-address is not working |
| ES3528MO-FLF-38-01098 | ES3528M about "no dot1q-tunnel system-tunnel-control" |
| ES3528MO-FLF-38-01099 | Can't set TACACS server key at WEB page |
| ES3528MO-FLF-38-01100 | A switch does not work after receiving several DHCPSNP attack messages in the syslog |
| ES3528MO-FLF-38-01101 | [ES3528M_V1.4.16.1] The class-map "match-any rule" debug message should not be displayed. |
| ES3528MO-FLF-38-01103 | When Radius server have two Dynamic-Vlan setting, ES3552M doesn't work dynamic-vlan in 1.4.16.1. |
| ES3528MO-FLF-38-01104 | <Romsat>strange record in the dhcp binding table. |
| ES3528MO-FLF-38-01109 | I have no permit for using cable-diagnostics test on Web. |
| ES3528MO-FLF-38-01110 | Login account with access level 7 and can't use "show cable-diagnostics" on console and telnet |
| ES3528MO-FLF-38-01111 | When we set the description to port by web with some rule , we will face error message. |
| ES3528MO-FLF-38-01114 | [ES3528M_V1.4.16.1] Mistake in reading information over SNMP - local VLANs are missing |
| ES3528MO-FLF-17-00074 | Fw: Zero MAC Filter |
| ES3528MO 1-FLF-38-00061 | Add "show ip igmp snooping vlan" command |
| ES3528MF-PoE-FLF-38-00011 | We can't get correct linking information on web panel. |
| ES3528MO-PoE-FLF-38-00024 | "no power inline compatible" does not take effect |
| ES3528M-SFP-FLF-38-00256 | [ES3528M-SFP] there is no any output message when showing interface transceiver |
| ES3528MO-FLF-38-01090 | ES3528M - doesn't work dhcp snooping with "information option" |
| ES3528MO-FLF-38-01116 | [ES3528M_V1.4.16.1] IGMP report lost problem. |
| ES3528MO-FLF-38-01113 | ?? BPDU Guard work (traffic doesn’t pass), but port status doesn’t change |

Runtime V 1.4.18.2 Diag V 1.2.1.3; Loader V1.0.4.1

| ES3528MO-FLF-38-00873 | There is nothing about "add-to-running-config" by snmp in the MIB file of ES3528M |
| --- | --- |
| ES3528MO-FLF-38-01092 | [ES3528M_V1.4.16.1] Switch is hanging when user issue clear ip igmp snooping groups dynamic |
| ES3528MO-FLF-38-01094 | [ES3528M_V1.4.16.1] No ip DHCP snooping verify mac-address is not working |
| ES3528MO-FLF-38-01098 | ES3528M about "no dot1q-tunnel system-tunnel-control" |
| ES3528MO-FLF-38-01099 | Can't set TACACS server key at WEB page |
| ES3528MO-FLF-38-01100 | A switch does not work after receiving several DHCPSNP attack messages in the syslog |
| ES3528MO-FLF-38-01101 | [ES3528M_V1.4.16.1] The class-map "match-any rule" debug message should not be displayed. |
| ES3528MO-FLF-38-01103 | When Radius server have two Dynamic-Vlan setting, ES3552M doesn't work dynamic-vlan in 1.4.16.1. |
| ES3528MO-FLF-38-01104 | <Romsat>strange record in the dhcp binding table. |
| ES3528MO-FLF-38-01109 | I have no permit for using cable-diagnostics test on Web. |
| ES3528MO-FLF-38-01110 | Login account with access level 7 and can't use "show cable-diagnostics" on console and telnet |
| ES3528MO-FLF-38-01111 | When we set the description to port by web with some rule , we will face error message. |
| ES3528MO-FLF-38-01114 | [ES3528M_V1.4.16.1] Mistake in reading information over SNMP - local VLANs are missing |
| ES3528MO-FLF-17-00074 | Fw: Zero MAC Filter |
| ES3528MO 1-FLF-38-00061 | Add "show ip igmp snooping vlan" command |
| ES3528MF-PoE-FLF-38-00011 | We can't get correct linking information on web panel. |
| ES3528MO-PoE-FLF-38-00024 | "no power inline compatible" does not take effect |
| ES3528M-SFP-FLF-38-00256 | [ES3528M-SFP] there is no any output message when showing interface transceiver |
| ES3528MO-FLF-38-01090 | ES3528M - doesn't work dhcp snooping with "information option" |
| ES3528MO-FLF-38-01116 | [ES3528M_V1.4.16.1] IGMP report lost problem. |
| ES3528MO-FLF-38-01113 | ?? BPDU Guard work (traffic doesn’t pass), but port status doesn’t change |
| ES3528MO-FLF-38-01115 | Exception messages will appear if ES3528M telnet to another one, show running. |
| ES3528MO-FLF-38-01095 | Wrong OID in traps |

Runtime V 1.4.16.2 Diag V 1.2.1.3; Loader V1.0.4.1

| ES3528MO-FLF-38-01092 | [ES3528M_V1.4.16.1] Switch is hanging when user issue clear ip igmp snooping groups dynamic |
| --- | --- |
| ES3528MO-FLF-38-01096 | adding lock unlock function at run-time, loader and Diag |

Runtime V 1.4.16.1 Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-01041 | network-access mode mac-authentication can not work on access port |
| --- | --- |
| ES3528MO-FLF-38-01040 | New feature request, show port security such as cisco. |
| ES3528MO-FLF-38-01059 | PVLAN is not working after changing the port speed duplex as 100fx then rebooting |
| ES3528MO-FLF-38-01051 | console is hanging after receiving signamax lldp packet |
| ES3528MO-FLF-38-01052 | there is an SNMP packet error when SNMPwalk the OID(1.0.8802.1.1.2.1.5.4795.1.2) |
| ES3528MO-FLF-38-01083 | Exception and web hang problem |
| ES3528MO-FLF-38-01087 | When enable global DHCP snooping ,client in the valn which disable DHCP snooping can't get ip |
| ES3528MO-FLF-38-00981 | Customer loss some configuration in suspend vlan after saved and rebooting ES3528MO. |
| ES3528MO-FLF-38-00972 | LBD: can not work |
| ES3528MO-FLF-38-01042 | hanging up if to press button "Enter" |
| ES3528MO-FLF-38-01081 | DHCP Option 82 Remote ID field is empty (FW 1.4.14.3) |
| ES3552MO-FLF-17-00064 | SNMP: SNMP Walk enterprises.202.20.66 stop then display error |

Runtime V 1.4.14.6 Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-001082 | If the HDCP client and server are in the the same Vlan, DHCP SNP and relay should still work at the same time |
| --- | --- |

Runtime V 1.4.14.5 Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00976 | When dhcp snooping work,dhcp relay function won't work well. |
| --- | --- |

Runtime V 1.4.14.4 Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00972 | LBD: can not work |
| --- | --- |
| ES3528MO-FLF-38-01056 | DHCPSNP option remote-id ip mode doesn't work |
| ES3528MO-FLF-38-00981 | Customer loss some configuration in suspend vlan after saved and rebooting ES3528MO. |

Runtime V 1.4.14.3 Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00965 | ES3528M - capabilities flowcontrol vanishes afters reboot |
| --- | --- |

Runtime V 1.4.14.2; Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00970 | [Enhancement] Extend MVR receiver group from 5 to 10 with mask. |
| --- | --- |
| ES3528MO-FLF-38-01028 | [ES3528MO_V1.4.14.1] DHCPSNP will block packets which the IP-in-IP packets related to DHCP |
| ES3528MO-FLF-38-01016 | [ES3528M_V1.4.14.1] crash problem |
| ES3528MO-FLF-38-00955 | When telnet from commander to candidate,console will be hank for 15-20 minutes. |
| ES3528MO-FLF-38-00964 | ES3528M: Telnet through the switch with wrong IP , the the VTY got freezed |
| ES3528MO-FLF-38-00963 | Web page for function of blocking IGMP query packets per VLAN configuration. |
| ES3528MO-FLF-38-00873 | There is nothing about "add-to-running-config" by snmp in the MIB file of ES3528M |
| ES3528MO-FLF-38-01022 | [Enhancement] Add new CLI command to show the dropped Mac address |
| ES3528MO-FLF-38-00969 | The config on LACP can't type in running table. |
| ES3528MO-FLF-38-01025 | when we want to get ip source-gurad information by snmp , we will face error message) |
| ES3552MO-FLF-38-00620 | Fix customer issue: packets larger than 536 bytes lost on egress rate-limit enabled port |

Runtime V 1.4.14.2; Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00965 | ES3528M - capabilities flowcontrol vanishes afters reboot |
| --- | --- |

Runtime V 1.4.14.1; Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00957 | ES3528M - capabilities flowcontrol vanishes afters reboot |
| --- | --- |
| ES3528MO-FLF-38-00936 | RFE: add port speed and duplex into port link status (LOG) DocNo : 38-2009-03-02 |
| ES3528MO-FLF-38-00974 | Enhancement: CLI show more than one page, need to provide guideline |
| ES3528MO-FLF-38-00971 | [ES3528M_V1.4.12.0] Port Security action will not change Port Admin status. |
| ES3528MO-FLF-38-00966 | If we set the unknown unicast packet-rate,it will effect all of unicast packets. |
| ES3528MO-PoE-FLF-17-00144 | SMC6152PL2 - Cannot see sFlow traffic f/w v1.4.8.4 |
| ES3528MO-FLF-38-00866 | ES3528M - Login one times with WEB UI ,will display many entries in ”sh log ram“ |
| ES3528MO-FLF-38-00980 | Cluster problem, Web management can not connect to member. |

Runtime V 1.4.12.3; Diag V 1.2.1.1; Loader V1.0.4.0

| ES3528MO-FLF-38-00968 | MacAuth: Enable MacAuth failed if switchport mode is access mode. |
| --- | --- |

Runtime V 1.4.12.2; Diag V 1.2.1.1; Loader V1.0.4.0

| ES3552MO-FLF-38-00618 | [enhancement] extend the neworl-access black list entry from 10 to 32 |
| --- | --- |

V 1.4.12.1

| ES3528MO-FLF-38-00962 | [ES3528M_V1.4.10.1] show ip igmp snooping groups by Web problem |
| --- | --- |
| ES3528MO-FLF-38-00960 | ES3528M pvlan problem |
| ES3528MO-FLF-38-00958 | (Enhancement) -- IP IGMP Query-drop vlan<VID> |
| ES3528MO-FLF-38-00956 | Mac-vlan, tagged packet will be modify by mac-vlan agin |
| ES3528MO-FLF-38-00954 | The error message in cluster backdoor. |
| ES3528MO-FLF-38-00953 | can’t save running-configuration after upgrading |
| ES3528MO-FLF-38-00948 | Cable diagnostics problem of Pair A & Pair B |
| ES3528MO-FLF-38-00946 | [ES3528M_V1.4.10.1] Console will freezing after re-configureing radius-server timeout/retransmit/key |
| ES3528MO-FLF-38-00945 | [ES3528M_1.4.10.1] MSTP instances can not be read by SNMP |
| ES3528MO-FLF-38-00944 | ES3528M: After reload a switch a command “no negotiation” is lost on combo port with media-type sfp- forced 100fx. |
| ES3528MO-FLF-38-00937 | RFE: ES3528M enlargement of messages length stored in LOG DocNo : 38-2009-03-08 |
| ES3528MO-FLF-38-00936 | RFE: add port speed and duplex into port link status (LOG) DocNo : 38-2009-03-02 |
| ES3528MO-FLF-38-00933 | (Enhancement) -- customer asked for a function to deny certain MAC |
| ES3528MO-FLF-38-00930 | (Enhancement) -- block IGMP query packets |
| ES3528MO-FLF-38-00929 | add port speed and duplext info in log when port links up |
| ES3528MO-FLF-38-00928 | (Enhancement) -- add port seepd and duplex info into log when interface links up |
| ES3528MO-FLF-38-00919 | (Enhancement) -- ES3528M - clear ip source guard binding table |
| ES3528MO-FLF-38-00913 | Time-based ACL web page added |
| ES3528MO-FLF-38-00912 | PPPoE-IA Web page added |
| ES3528MO-FLF-38-00901 | DHCP packets wil modify Vlan after trapped to CPU if Mac Valn is enabled |
| ES3528MO-FLF-38-00900 | (Enhancement) -- function management by https on ES3528M |
| ES3528MO-FLF-38-00863 | ES3528M: Upgrade opcode is requesting different filename |
| ES3528MO-FLF-38-00859 | ES3528M: Misspeling in console timeout |
| ES3528MO-FLF-38-00856 | ES3528M: CPU(memory) rising/falling thresholds are missing in running config |
| ES3528MO 1-FLF-38-00046 | problem ES4612( Ping dut loss after boot dut many hours later. PC still can ping PC) snmp server /snmp walk |
| ES3528MO 1-FLF-38-00044 | uplink-to-uplink command is ambiguous with uplink argument |
| ES3528MO 1-FLF-38-00043 | MVR: Set port mode access, mvr receiver port still active |
| ES3528MO 1-FLF-38-00047 | After change unit more times in cluster on ES3528MO, we will lose the managment. |
| ES3528MO 1-FLF-38-00040 | ip igmp snooping query-drop loss displaing per port configuration |

V 1.4.10.1

| ES3528MO-FLF-38-00821 | ES3528M - "spanning-tree loopback-detection action" in WEB |
| --- | --- |
| ES3528MO-FLF-38-00925 | Management telnet problem |
| ES3528MO-FLF-38-00935 | We can't set the packet-rate in unicast in FW 1.4.8.12. But we can set the same step in FW 1.4.8.11. |
| ES3528MO 1-FLF-38-00040 | ip igmp snooping query-drop loss displaing per port configuration |
| ES3528MO-FLF-38-00858 | clear arp-cache via SNMP on ES3528M priority low |

| ES3528MO-FLF-38-00861 | ES3528MO-FLF-38-00861 |
| --- | --- |
| ES3528MO-FLF-38-00864 | Show wrong in media-type sfp-forced |
| ES3528MO-FLF-38-00865 | The time-range can't disable after command of ACL |
| ES3528MO-FLF-38-00872 | fiber ports 25 and 26 and shutdown ES3528M fw 1.4.8.0 |
| ES3528MO-FLF-38-00877 | No option of pvlan in web interface. |
| ES3528MO-FLF-38-00903 | [ES3528M_1.4.8.2] There is an output format error about "show access-list tcam-utilization" via telnet |
| ES3528MO-FLF-38-00904 | "power inline overload-auto-recover" cli prompt message |
| ES3528MO-FLF-38-00905 | If user do not assign a specific name of MSTP, then you will see the wrong encoding or decoding characters in running-config |
| ES3528MO-FLF-38-00919 | ES3528M - clear ip source guard binding table |
| ES3528MO-FLF-38-00928 | add port seepd and duplex info into log when interface links up |
| ES3528MO-FLF-38-00930 | block IGMP query packets |
| ES3528MO-FLF-38-00932 | ES3528M: remove the CLI of "no ip igmp snooping" which is not a valid command for a L2 switch |
| ES3528MO-FLF-38-00933 | customer asked for a function to deny certain MAC |
| ES3528MO-FLF-38-00934 | Fix the possible exception caused by many users execute show log command concurrently |
| ES3552MO-FLF-38-00617 | LLDP can only show 1 entry in detailed |
| ES3528MO-FLF-17-00071 | [Port security]: When a static security mac is set on a diamond port, no trap sent when the mac comes from |
| ES3552MO-FLF-17-00062 | enhance:igmpsnp and mvr default forward priority is disabled |
| ES3528MO 1-FLF-38-00031 | can't get information from some OIDs (for mac-filtering) on ES3528M |

V 1.4.8.12

| ES3528MO 1-FLF-38-00038 | when booting igmpsnp will cause exception |
| --- | --- |
| ES3528MO 1-FLF-38-00037 | mac-valn paremeter mask put at wrong position |

V 1.4.8.11

| ES3528MO-FLF-38-00922 | Argument out do not initial. |
| --- | --- |
| ES3528MO 1-FLF-17-00129 | report won't be trap to cpu when there are dynamic entry in chip |

V 1.4.8.10

| ES3528MO 1-FLF-17-00128 | igmp packet won't be trap to cpu on receiver port when enable mvr |
| --- | --- |

Runtime V 1.4.8.9; Diag V 1.2.1.1; Loader V1.0.3.0

| ES3528MO-FLF-17-00066 | Latest SMC phase-4 17 image V1.4.8.0 SFP panel display issue. |
| --- | --- |
| ES3528MO-FLF-38-00806 | ES3528M, ES3526XAv2 - OID for viewing "show mac-address-table count" |
| ES3528MO-FLF-38-00877 | No option of pvlan in web interface. |
| ES3528MO 1-FLF-38-00030 | MVR: mvr source port won't forward report |
| ES3528MO 1-FLF-38-00032 | MVR: add mvr group range configuration |
| ES3528MO 1-FLF-38-00033 ES3528MO-FLF-38-00910 | enhance mac-base vlan support mask CLI: Remove the global varibale to each task. |

V 1.4.8.8

| ES3552MO-FLF-38-00908 | Syslog : memory corruption |
| --- | --- |

V 1.4.8.7

| ES3552MO-FLF-38-00890 | CLI exception |
| --- | --- |
| ES3552MO-FLF-38-00879 | [ES3528M] Syslog format problem |
| ES3552MO-FLF-38-00884 | [ES3528M] Syslog format problem |
| ES3552MO-FLF-38-00880 | . [ES3528M] Management access |
| ES3552MO-FLF-38-00870 | Traffic Segmentation: wrong CLI command: uplink-to-uplink |
| ES3552MO-FLF-38-00875 | [ES3528M] snmpwalk lldp remote devices problem on ES3528M |

V 1.4.8.6

| ES3528MO-FLF-38-00878 | ES3528M - static mac records disappear when port security is enabled |
| --- | --- |
| ES3528MO-FLF-38-00874 | 轉寄: [ES3528M] MSTP Question |
| ES3528MO-FLF-38-00806 | ES3528M, ES3526XAv2 - OID for viewing "show mac-address-table count" |
| ES3528MO-FLF-38-00724 | [ES3528MO_V1.3.7.4] SNMP staPortTable objects problem, about RootGuard, BpduGuard, BpduFilter |

V 1.4.8.5

| ES3528MO-FLF-17-00069 | SMC6128L2 mac-address |
| --- | --- |

V 1.4.8.4

| ES3528MO-FLF-17-00067 | The CLI of 38 copy to 17 |
| --- | --- |

V 1.4.8.3

| ES3528MO-FLF-38-00882 | Enhance Modify super password to MD5+Randon key |
| --- | --- |

V 1.4.8.2

| ES3528MO-FLF-38-00876 | CLI command losing priority of administrator and normal users |
| --- | --- |

V 1.4.8.1

| ES3528MO-FLF-38-00813 | Web pages of configuable DHCP option 82 |
| --- | --- |
| ES3528MO-FLF-38-00868 | add a new CLI to disable option 82 to carry sub-type of CIR & RID |
| ES3528MO-FLF-38-00869 | add a new mib node to disable option82 to carry sub-type of CID & RID |
| ES3528MO-FLF-38-00870 | Traffic Segmentation: wrong CLI command: uplink-to-uplink |

V 1.4.8.0

| ES3552MO-FLF-38-00486 | How to manage the LLDP via SNMP for ES3528M |
| --- | --- |
| ES3552MO-FLF-38-00524 | ProtocolVLAN:the members of protocol-vlan didn't receive packet when inject ip frame with vid=0. |
| ES3552MO-FLF-38-00565 | Date&time not saved during reboot |
| ES3552MO-FLF-38-00567 | Web: It doesn't provide "PoE" help page. |
| ES3552MO-FLF-38-00568 | PoE:Maximum power allocation range is not correct on web. |
| ES3552MO-FLF-38-00569 | Web :Edit classes on policy-map page without checking the check box, it will not show any pop-up message like "Remove Policy" |
| ES3552MO-FLF-38-00570 | ES3528M: DHCP Snooping – false bindings are shown on port 1 |
| ES3552MO-FLF-38-00575 | no permit/deny (MAC ACL) - missing [cos cos bitmask] option |
| ES3552MO-FLF-38-00577 | corrections to grammar in "show ip igmp snooping" |
| ES3552MO-FLF-38-00578 | correction to spelling convention in "show mvr" |
| ES3552MO-FLF-38-00580 | correction to grammar in "show mvr interface" |
| ES3552MO-FLF-38-00581 | giga-phy-mode command errors |
| ES3552MO-FLF-38-00582 | corrections to spelling convention in "show lldp config" output |
| ES3552MO-FLF-38-00584 | corrections to grammar in web ui - show lldp info remote device details |
| ES3552MO-FLF-38-00585 | ES3528M latest FW |
| ES3552MO-FLF-38-00586 | DAI crash after 30 minites or 1 hours |
| ES3552MO-FLF-38-00587 | DAI validation parameters on ES3528M priority low |
| ES3552MO-FLF-38-00588 | IGMPSNP:It should be able to create 64 static multicast group. not 16. |
| ES3552MO-FLF-38-00590 | output modifier added for command of" show interface brief" |
| ES3552MO-FLF-38-00592 | SFP in ports 28 on ES3528MO has ifType is wrong in sFlow counter packets. |
| ES3552MO-FLF-38-00593 | ES3528M, ES3526XAv2 - OID for viewing "show mac-address-table count" |
| ES3552MO-FLF-38-00594 | add CLI command to stop sending trap |
| ES3552MO-FLF-38-00595 | ES3528M: Copy startup-config running-config will cause skipping of opcode auto upgrade |
| ES3552MO-FLF-38-00596 | ES3528M-SFP-38 after create vlan 311 and copy startup,it will stop after reboot |
| ES3552MO-FLF-38-00597 | corrections to grammar/formatting in web ui |
| ES3552MO-FLF-38-00598 | BOOTP packet is blocked when DHCP snooping enable |
| ES3552MO-FLF-38-00599 | ES3528M DAI web management |
| ES3552MO-FLF-38-00600 | BOOTP packet is blocked when DHCP snooping enable |
| ES3552MO-FLF-38-00601 | ES3528M network-access RADIUS authorisation bug |
| ES3552MO-FLF-38-00602 | "BPDU tunneling" feature on Phase 4 firmware(v1.4.4.0) did not implemented completely as mentioned in spec. |
| ES3552MO-FLF-38-00603 | SSL and Port security causing web management problems and STP instability |
| ES3552MO-FLF-38-00604 | DNS: After replace the hostname setting, DUT can not show the correct configure. |
| ES3552MO-FLF-38-00605 | dot1qstaticunicaststatus and dot1qstaticunicastallowedtogoto will dispaer |
| ES3552MO-FLF-38-00573 | EAPS port just only 28 ports in port 52 switch. |

V 1.4.6.4

| ES3528MO-FLF-38-00619 | Port Lock |
| --- | --- |
| ES3528MO 1-FLF-38-00021 | 224.0.0.x can't be flood on vlan |

V 1.4.6.2

| ES3552MO-FLF-17-00060 | Makefile path error |
| --- | --- |

V 1.4.6.1

| ES3552MO-FLF-38-00735 | ES3528M: NTP has 24h offset |
| --- | --- |
| ES3552MO-FLF-38-00570 | ES3528M: DHCP Snooping – false bindings are shown on port 1 |
| ES3528MO-FLF-17-00064 | add Mac address info in the Trap message of port security when there is MAC violation |
| ES3528MO-FLF-38-00839 | can't login with default username/passwd |
| ES3528MO-FLF-38-00784 | ES3528M: Copy startup-config running-config will cause skipping of opcode auto upgrade |

V 1.4.6.0;

| ES3552MO-FLF-38-00486 | How to manage the LLDP via SNMP for ES3528M |
| --- | --- |
| ES3552MO-FLF-38-00519 | L2PT:Can't find L2PT function via mib browser. |
| ES3552MO-FLF-38-00525 | RadiusAcc:The total session should be 4 , not 8 when opening by using telnet and SSH at the same time |
| ES3552MO-FLF-38-00531 | diffserv:The packet forwarding ratio doesn't correspond to the cos. |
| ES3552MO-FLF-38-00538 | ES3528M: help with diagnosis of management freezing |
| ES3552MO-FLF-38-00540 | ES3552M do not Telnet access. Display "memPartAlloc: block too big" message From console. |
| ES3552MO-FLF-38-00542 | 轉寄: Why why ith the status of Option82 is Disable ? |
| ES3552MO-FLF-38-00543 | Trunk:Create a static trunk,commmand "sh run" ,it displays "lacp" word in running config. |
| ES3552MO-FLF-38-00544 | PoE Web pages are gone |
| ES3552MO-FLF-38-00545 | Enable igmp snooping on specify vlan, unknown multicast data is still flood |
| ES3552MO-FLF-38-00546 | "no ip http secure-server" command will cause switch to reboot automatically |
| ES3552MO-FLF-38-00547 | backdoor passwd should be encrypted |
| ES3552MO-FLF-38-00550 | description of vlan display incorrect while create mst instance |
| ES3552MO-FLF-38-00552 | vlantrunk:vlantrunk config on lacp will be lost after DUT reload |
| ES3552MO-FLF-38-00554 | Rate limit configuration will casue the error message "Failed to join egress list on ethernet 1/1" during provision |
| ES3552MO-FLF-38-00556 | ifType of SFP ports 28 on sFlow counter packets is not correct in ES3528MO |
| ES3552MO-FLF-37-00557 | [ES3528M] Unknown unicast storm control |
| ES3552Mo-FLF-38-00558 | PPPoE should not replace the tag if the packets came from a trusted port |
| ES3552MO-FLF-38-00559 | ES3528M - IF-MIB::ifDescr is empty |
| ES3552MO-FLF-38-00560 | Port link status will not be up after run TDR diagnostics test. |
| ES3552MO-FLF-38-00561 | [ES3528MO-PoE] The ifOperStatus of networkAccessPortLinkDetectionTrap is always up. |
| ES3552MO-FLF-38-00562 | miss the command of " show ip igmp snooping groups" on Phase4 image |
| ES3552MO-FLF-38-00563 | ES3528M cableDiag question |
| ES3552MO-FLF-38-00564 | Suspended Vlan 4093 will become active after reload the device |
| ES3552MO-FLF-38-00565 | Date&time not saved during reboot |
| ES3552MO-FLF-38-00566 | certain 'name-server' command doesn't apply on ES3528M priority low |
| ES3528MO 1-FLF-38-00016 | ES3528M reboots on FW 1.4.4.0 priority medium |
| ES3528MO 1-FLF-38-00015 | Suspended Vlan 4093 will become active after reload the deviceES3528MF-PoE-FLF-38- |
| ES3528MF-PoE-FLF-38-00007 | The NTP config didn't supported ntp server "version" and "poll" in 1.3.7.4 and 1.4.4.0 |

V1.4.4.0

1. Support Software watch dog

2. Support EPAOL pass through

3. Support MAC address notification

4. Support Cable Diag

5. Support Telnet backdoor

6. Support Per vlan IGMP snooping

7. Support IPv6 MLD snooping

8. Support L2 protocol tunnel

9. Support MDIX forced mode

V1.4.2.1

1. Support EAPOL forwarding/transparent (New CLI command: “Console(config)#[no] dot1x eapol-pass-through”. Default: Disabled)

2. Support VLAN trunking on FE ports (Ever GE ports only)

3. Support up to 4K (4092) VLANs

4. Support Time-based ACL

5. Support PPPoE Intermediate Agent

6. Support Individual traffic controls for Rate-limiting, Broadcast, Multicast, and UnknownUnicast storms

7. Support DHCP Relay Option 82

8. Support new Marvell 88E6097 (Opal+), as well as existing 88E6095 (Opal), chips

9. Support DHCPSNP: log/trap for the mac address of a rouge server, together with the receive port.

10. Enlarge DHCPSNP binding entries per port up to 16

11. Support up to 8 telnet sessions

12. Support show version" displays the information of "Chip Device ID" with ‘88E6097’ or ‘88E6095

13, Support System resources, CPU utilization and memory status, on the WebGUI and SNMP

14. Support ACL profile for dynamic QoS assignment, a.k.a. auto security

15. Support Port Utilization (kbits/sec, Pkts/sec, % Utilization in recent 300 seconds)

16. Support to save the startup configuration file for that being downloaded by dynamic provision (via DHCP option 66,67)

17. Support Make DHCP option 82 remote ID configurable for MAC or IP. (New CLI command: “Console(config)#[no] ip dhcp relay information option remote-id {mac ¦ ip}

V1.3.2.0

1. Support DNS

modifications:

V1.3.3.1

1. ES3528M-SFP-FLF-38-00214: MAC address that connects to promiscuouse port will be learned in wrong interface because of Windows XP bug

V1.3.3.0 is same as V1.3.2.1

V1.3.2.1 fix the following:

1. ES3528M-SFP-FLF-38-00202: MulticastStorm: settings have no effect if the cable is reconnected on 100M speed.

2. ES3528M-SFP-FLF-38-00206: IPConfiguration: telnet port can be overlapped with HTTP server port number

3. ES3528M-SFP-FLF-38-00207: DNS:CLI lossing command "ping dns.ntc.com.tw"

4. ES3528M-SFP-FLF-38-00208: DHCPDynamicProvision:The DHCPDynamicProvision with DNS is not working.

V1.3.2.0 fix the following:

1. ES3528M-SFP-FLF-38-00196: IGMP-FILTER: DUT doesn't display IGMP Profile information correctly in running configuration file.

2. ES3528M-SFP-FLF-38-00201: UserAuth-The command "enable" doesn't work with keying in valid user name and passward to log in to privilege access level.

3. ES3528M-SFP-FLF-38-00203: PrivateVLAN:It will display error msg after indiscriminate setting.

Known Bugs and Issues:

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

1. If this switch is configured flow control at specific giga port and inject broadcast packet or unknown unicast packet to this port then it will send out pause frame out due to the switch HW design is cascading MAC(Diamond) connected to three other MAC (opal) which are always link up.

2. the combo port 25 ~28 and front port 1 ~24 are all connected with Dx107 chip and the SDK configure all Dx107 ports to join all vlans thus it will flood all broadcast or unknown unitcast packet to front ports from combo ports even though the user port does not join the same vlan as combo ports and occupy buffer memory and cause packet loss at front ports

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2007 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ES3528M-SFP Private Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| ES3528M-SFP.mib | 20061206001 |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Customer Support:

By Phone:

By Email:

By Web:

By Fax: