Fast Ethernet Switch ES4626F/ES4650F

Phase 2 Firmware Version 1.2.4.6

INTRODUCTION:

The ES4626F/ES4650FD is a 26/50-port 10/100/1000 L3 stackable management Gigabit Ethernet Switch. The switch configuration is 24/48 10/100/1000 Base-T ports and 4 combo ports at front panel, which are comprised of an RJ45 and a SFP. 2 10G optional modules and stacking ports in Rear panel.

The base hardware is a 1U height, 19” rack mountable metal enclosure. The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RJ-45 RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common HTTP browsers such as Netscape Navigator (Version 6.2 or above) or Microsoft’s Internet Explorer (Version 5.0 or above)

Local Console Management (LCM) allows the user to monitor and configure the ES4626F/ES4650F from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

It is recommended that one thoroughly review this release notes prior to the installation or upgrade of this product.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Version with Phase 1 feature | 1.1.0.1 | Test | 2009/03/11 |
| Patch Version | 1.1.0.2 | Test | 2009/3/13 |
| PR Update Version | 1.1.0.3 | Update | 2009/5/4 |
| Test Version | 1.1.1.0 | NTC Sys & Func test | 2009/5/8 |
| Test Version | 1.1.2.0 | Bug regress + ACT | 2009/6/10 |
| Test Version | 1.1.3.0 | Bug regress + ACT | 2009/6/12 |
| Test Version | 1.2.1.0 | QA Sys & Func test | 2009/6/18 |
| Test Version | 1.2.2.0 | NTC test | 2009/7/14 |
| Test Version | 1.2.3.0 | NTC test | 2009/9/15 |
| Current Version | 1.2.4.0 | MP | 2009/9/28 |
| Patch version | 1.2.4.6 | Patch The same fix in -17 version | 2009/12/15 |

3LOADER:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Disposed Version | 1.1.0.1 |  | 2009/03/11 |
| Disposed Version | 1.1.0.2 |  | 2009/5/4 |

DIAG:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Disposed Version | 1.1.0.1 |  | 2009/03/11 |
| Disposed Version | 1.1.0.2 |  | 2009/5/4 |
| Disposed Version | 1.1.0.3 |  | 2009/5/8 |
| Test Version | 1.1.0.4 |  | 2009/8/25 |

HARDware compatibility:

ALL

BootPROM compatibility:

ALL

Network Management Software Support:

| NMS Platform | Version No. | Module No. |
| --- | --- | --- |
| TBD |  |  |

If you install this image, you may not have control of all of the latest features of this product until the next version(s) of network management software. Please review the software release notes for your specific network management platform for details.

SUPPORTED FUNCTIONALITY:

| Accton Product Feature List |  |  |
| --- | --- | --- |
| Product Configuration |  | 20GT+4ComboG+2XG |
| Switch ASIC |  | BCM56504 |
| PHY |  | Broadcom BCM5464R*5 +BCM5464SR*1 |
| Flash Memory |  | 64MB |
| SDRAM |  | 256MB |
| CPU |  | 8248/400Mhz |
| L2 MAC Table |  | 16K |
| L3 IPv4 Host Table |  | 2k |
| L3 IPv4 Net Table |  | 256 entries |
| Standalone/Stackable |  | Stackable |
| FP Field Selector |  | 16 selectors |
| FP Rules |  | 2k |
| Integrated Packet Buffer Memory |  | 2MB |
| Number of Units in the stack |  | 8 |
| PoE Module |  | NO |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | YES 1) Total 32 Trunk (Including Cisco EtherChannel like trunks and 802.3ad) 2) 2~8 port/trunk for GE 3) 2~4port/trunk for 10GE |
|  | Cisco EtherChannel Liked | YES 1) Total 32 Trunk (Including Cisco EtherChannel like trunks and 802.3ad) 2) 2~8 port/trunk for GE 3) 2~4port/trunk for 10GE |
|  | Unicast / Multicast traffic Balance over Trunking port | YES, Six Trunk Balance mode on Unicast 1. SA, VLAN, Ethertype, source module ID and port ID 2. DA, VLAN, Ethertype, source module ID and port ID 3. SA xor DA, VLAN, Ethertype, source module ID and port ID 4. SIP and source TCP/UDP port 5. DIP and destinat |
| VLAN | IEEE 802.1Q Tagged Based | YES |
|  | Port-Based | YES 1) L2 supports 4094 VLAN 2) L3 supports 4094 VLAN |
|  | Private VLAN | YES (Traffic segmentation) |
|  | GVRP | YES (256 VLAN) |
|  | 802.1v Protocol | YES 1) 16 entries per port 2) VLAN group by (Frame type, Protocol) - 3 Frame type : Ethernet type II, 802.3 LLC, 802.3 SNAP - Protocols : IPv4, IPv6, IPX-RAW, IPX-LLC,…other |
| Spanning Tree | IEEE 802.1D | YES |
|  | IEEE 802.1s | YES (32 instance) |
|  | IEEE 802.1w | YES |
|  | Spanning Tree Fast Forwarding | YES (per port configure) |
| BroadCast Strom Control |  | YES (per port configure) 1) per Port Rate control/limit; 2) range : 500 ~ 3FFFF packets/sec 3) resolution: 1 packet/sec Note : starting from 500 packets/sec and increament at 1 packet/sec up to 3FFFF packets/sec |
| IGMP Snooping | IGMP Snooping v1/v2 | YES (255 Groups for Static/dynamic, 32 router port) |
|  | IGMP v1/v2 querier support | YES |
|  | IGMP Immediate Leave | YES |
|  | IGMP throttling |  |
|  | IGMP Filtering | NO |
|  | IFMPSNP Leave Proxy | NO |
|  | IGMPV3 Snooping | YES(Phase3) |
| MVR |  | NO |
| Jumbo Frame |  | 9K |
| QinQ |  | YES |
| LLDP |  | YES |
| QoS Features |  |  |
| Number of pripority queue |  | 8 |
| Scheduling for pripority queue | WRR Pripority scheduling | YES (per port configure) 1) weight value range 1-15 2) default weight ratio 1:2:4:6:8:10:12:14 |
|  | Strict Pripority scheduling | YES |
|  | Hybrid (WRR + Strict) Pripority scheduling | YES |
| COS | 802.1p based COS | YES (per system configure) |
|  | IP TOS Precedence based COS | YES (per system configure) |
|  | IP DSCP based COS | YES (per system configure) |
|  | TCP/UDP Port based COS | YES (per system configure) |
| Rate Limiting | Ingress | YES 1) Per Port-based 2) Giga: - Range 1M bps - 1G bps - Resolution 1M bps 3) 10 Giga: - Not support |
|  | Egress | YES 1) Per Port-based 2) Giga: - Range 1M bps - 1G bps - Resolution 1M bps 3) 10 Giga: - Not support |
| DiffServ |  | YES |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | YES 1) Per system/stack : Total 1024 MAC 2) Per port : upto 1024 MAC 3) auto-learned support |
| Dynamic Port Security (MAC-based) |  | Next Release |
| 802.1x | Port-Based | YES 1) support both Single-host/ Multi-host mode 2) Single-host mode: - Per port 1 MAC 3) Multi-host mode: - Per port : upto 1024 MAC - Per system/stack : Total 1024 MAC) |
|  | Mac-Based | Next Release |
|  | Supplicant Support | YES 1) support Win XP 2) support Win 2000 w/ SP4 3) support Win 95/98 w/ AEGIS |
|  | Encryption supported | YES Types supported MD5 TLS,TTLS, PEAP |
| ACL | Number of rules | 96 MAC ACE, 96 IPv4 ACE |
|  | L2/L3/L4 | YES 1. L2 SA/DA MAC, VLAN based 2. L3 SIP/DIP, subnet based 3. L4 TCP/UDP port |
| RADIUS authentication |  | YES 1) encryption : MD5, TTLS, PEAP, TLS 2) Multiple Radius Server : 5 servers |
| TACACS+ Authentication |  | YES 1) encryption : MD5, TTLS,PEAP,TLS 2) Multiple Radius Server : 1 servers |
| TACACS+ 3.0 |  | NO |
| MAC Based Authentication |  | NO |
| HTTPS and SSL (Secured Web) |  | YES supported cipher : 3DES, RC4 Digest: MD5, SHA 1, Public key: RSA |
| SSH 1.5/V2.0 (Secured Telnet session) |  | YES supported cipher : 3DES Digest: MD5, SHA 1, Public key: RSA |
| User name password authentication | Local Authentication | YES Cisco-like (5 User/password) |
|  | Remote Authentication via RADIUS/TACACS+ | YES Cisco-like |
| Management Interface Access Filtering | SNMP | YES |
|  | WEB | YES |
|  | TELNET | YES |
| Management Features |  |  |
| Cisco Like CLI |  | YES |
| Web Based Management |  | YES |
| Telnet |  | YES (4 Sessions) |
| Software Download/Upgrade | TFTP | YES (runtime only) |
|  | Xmodem | YES (Boot code only) |
| Dual Image |  | YES |
| Configuration Download/Upload | TFTP | YES (multiple copies) |
| SNMP | v1 | YES (5 SNMP community string; 5 trap receivers) |
|  | v2c | YES (5 SNMP community string; 5 trap receivers) |
|  | v3 | YES |
| RMON | RMON I (1,2,3,9 group) | YES 1) Statistic : Auto create, per port 1 entry 2) Histroy : Auto create, per port 2 entries 3) Alarm : Auto create, per port 1 entries 4) Event : per port 1 entry in average Note: RMON configure will not be saved to Flash) |
| BOOTP | Client | YES |
|  | Relay | NO |
| DHCP | Client | YES |
|  | Relay | YES (5 Entries/VLAN) |
|  | Snooping | NO |
|  | Option 82 Relay | NO |
|  | Server | YES (4 DHCP Pool number; 512 Entries per pool) |
| IP Source Guard |  | NO |
| Port Mirroring |  | YES 1) Support cross chip Port Mirroring 2) 1 mirror session: 1 to 1 port monitor on Giga port, 10G port can mirror multiple Giga ports 3) Support Ingress and/or Egress mirror |
| Event/Error Log | Local Flash | YES (4000 entires) |
|  | Remote server via System Log (RFC3164) | YES Cisco-like (5 Server) |
|  | SMTP | YES (3 Server) |
| DNS | Client | YES |
|  | Proxy | YES |
| Remote Ping |  | YES |
| SNTP |  | YES (5 Server, user configurable) |
| IP Clustering |  | NO |
| IPv6 Features |  |  |
| IPv4/IPv6 Dual Protocol Stack |  | YES |
| IPv6 Address Types | Unicast | YES |
|  | Multicast | YES |
| ICMPv6 |  | YES |
| ICMPv6 Redirect (Host) |  | YES |
| IPv6 Path MTU Discovery |  | YES |
| IPv6 Neighbor Discovery | Router Discovery | YES |
|  | Duplicate Address Detection | YES |
|  | Static Cache Entry | YES |
|  | Parameter Discovery | YES |
|  | Prefix Discovery | YES |
|  | Address Resolution | YES |
|  | Unreachable Neighbor Detection | YES |
|  | Redirect (Host) | YES |
| Stateless Autoconfiguration |  | YES |
| Manual Configuration |  | YES |
| Generic Prefix |  | YES |
| SNMP over IPv6 |  | YES |
| HTTP over IPv6 |  | YES |
| SSH over IPv6 |  | YES |
| IPv6 Telnet Support |  | YES |
| IPv6 DNS Resolver |  | YES |
| IPv6 RADIUS+ Support |  | YES |
| IPv6 TACACS+ Support |  | NO |
| IPv6 Syslog Support |  | YES |
| IPv6 SNTP Support |  | YES |
| IPv6 SMTP Support |  | YES |
| IPv6 TFTP Support |  | YES |
| Remote IPv6 Ping |  | YES |
| IPv6 QoS |  |  |
| IPv6 ACL | Number of rules: | 96 IPv6 ACE |
|  | L2/L3/L4 | YES 1. L2 SA/DA MAC, VLAN based 2. L3 SIP/DIP, subnet based 3. L4 NextHeader, TCP/UDP port, Flow Label |
| IPv6 DSCP remapping CoS |  |  |
| Layer 3 Features IPv4 |  |  |
| Host Table |  | 2k |
| Static Route |  | YES (64Entries) |
| Net Table |  | 256 entries (Dynamic 160, Static 64, RIF 32) |
| Number of IP interface |  | 32 |
| Multi-netting |  | YES |
| CIDR |  | YES |
| Unicast Routing | RIP v1/v2 | YES |
|  | OSPF | YES |
| ARP |  | YES (Support 256 static ARP configuration and Up to 4K ARP Entries) (For Dynamic 3792, Static 256, RIP 32, Vrrp 16) |
| Proxy ARP |  | YES |
| Multicast Routing | Multicast groups | NO |
|  | IGMP v1/v2 | NO |
|  | DVMRP | NO |
|  | PIM-DM | NO |
|  | PIM-SM | NO |
| IP Redundancy | VRRP | YES |
| Optioal Module List |  |  |
| EM3626A-1FX-SC-S | 1-port 100BASE-FX 1) SC connector 2) SMF |  |
| EM3626A-1FX-SC-M | 1-port 100BASE-FX 1) SC connector 2) MMF |  |
| EM3626A-1SX-SC-M | 1-port 1000BASE-SX 1) SC connector 2) MMF |  |
| EM3626A-1LX-SC-S | 1-port 1000BASE-LX 1) SC connector 2) SMF |  |
| EM3626A-1GT-RJ45 | 1-port 1000BASE-T 1) RJ45 connector |  |
| EM3626A-1GX-GBIC-M | 1-port 1000Base-X 1) GBIC connector |  |
| EM4626H-XG-XFP |  | Yes |
| EM4626H-XG-10GT |  | YES |

Installation and Configuration Notes:

In general, the ES4626F/ES4650FD Switch will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing ES4626F/ES4650FD Switch, please follow the TFTP download instructions that are included with your firmware image upgrade kit.

Firmware Changes and Enhancements:

The following Known Issues have been fixed in this release of firmware.

Diag V1.1.0.3

| 1 | ES4626F-FLF-38-00031 | [Diag] The led of 10G module always trun off on ES4650F board. |
| --- | --- | --- |

Diag V1.1.0.4

| 1 | ES4626F-FLF-38-00129 | Diag wirespeed 10/100/1000 has the same speed, and got strange counter. LED got the same color. |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00130 | Diag wirespeed can not test mini Gbic ports. |

Runtime V1.1.0.1

| 1 | None | None |
| --- | --- | --- |

Runtime V1.1.0.2

| 1 | ES4626F-FLF-38-00008 | Modify MIB OID |
| --- | --- | --- |

Runtime V1.1.0.3

| 1 | ES4626F-FLF-38-00013 | Update web help page to firebolt version |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00015 | If DUT doesn't support PoE, poe TLV of LLDP shall be removed. |
| 3 | ES4626F-FLF-38-00017 | CLI_PortMirror : port mirror configuration will disappear after reloading |
| 4 | ES4626F-FLF-38-00021 | Login : The default value of login timeout is disabled. |
| 5 | ES4626F-FLF-38-00022 | IGMP Immediate Leave SNMP don't work, but others like igmpSnoopStatus, igmpSnoopQueryCount work well. |
| 6 | ES4626F-FLF-38-00009 | Logging Sendmail: Set the same sendmail host two times, it causes to show infinitely duplicate SMTP Servers. |
| 7 | ES4626F-FLF-38-00016 | Failed to set summer-time offset (0). |

Runtime V1.1.1.0

| 1 | ES4626F-FLF-38-00026 | TFTP_Auo : Editing the file --> TFTP?config file upload???? |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00024 | RMON_Auto : Alarm with Absolute Value ?????Alarm. |
| 3 | ES4626F-FLF-38-00018 | CLI_IP : Can't execute "no ip dhcp excluded-address x.x.x.x" command |
| 4 | ES4626F-FLF-38-00032 | The led of 10G module can not turn on or got a different color. |

Runtime V1.1.2.0

| 1 | ES4626F-FLF-38-00031 | [Diag] The led of 10G module always trun off on ES4650F board. |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00040 | IPv6-mgmt: Syslog host address can't be set to link-local adddress |
| 3 | ES4626F-FLF-38-00041 | IPv6-mgmt: SNTP server address, SMTP sendmail host address and TFTP address can't be set to link-local address |
| 4 | ES4626F-FLF-38-00025 | RMON_Auto : Alarm with Delta Value ?History entry?????. |
| 5 | ES4626F-FLF-38-00011 | Add "wire speed" test function |
| 6 | ES4626F-FLF-38-00002 | loader & diag. prepare |
| 7 | ES4626F-FLF-38-00062 | L2Switch:Flowcontrol doesn't work when stacking DUTs and receiver and sender in different DUT. |
| 8 | ES4626F-FLF-38-00012 | Move loader and diag build environment in ES4626F-FLF-38 folder. |
| 9 | ES4626F-FLF-38-00047 | CLI: The "show ip redirects" command does not display correctly |
| 10 | ES4626F-FLF-38-00078 | Stacking: runtime v1.1.1.0 cannot stack with v1.1.0.2 |
| 11 | ES4626F-FLF-38-00076 | IPv6AddrCfg: The default gateway may not be set to link-local addresses |
| 12 | ES4626F-FLF-38-00049 | Stacking: after the stack members are renumbered, connecting slave switch will cause loopback storming |
| 13 | ES4626F-FLF-38-00046 | Stacking: when multiple DUTs are stacked, setting boot configuration may cause the CLI to hang |
| 14 | ES4626F-FLF-38-00051 | Modify L3 table size for ES4626F. HOST_ROUTE 2K, NET_ROUTE 256 |
| 15 | ES4626F-FLF-38-00053 | Diffserv : the description of class-map shall not be set up to maximum 200 characters. |
| 16 | ES4626F-FLF-38-00054 | Diffserv : the description of policy-map can't be setup to maximum 200 characters. |
| 17 | ES4626F-FLF-38-00048 | CLI: the "show lldp info stat" command does not display stacked ports correctly |
| 18 | ES4626F-FLF-38-00045 | CLI: the "sh int sta brief" command does not display correctly when there are trunk members |
| 19 | ES4626F-FLF-38-00077 | Recurring mode of summer time page doesn't work. |
| 20 | ES4626F-FLF-38-00063 | CLI: It misspell the two word "september" about calendar command |
| 21 | ES4626F-FLF-38-00064 | CLI:The description of the snmp inform timeout is not correct |
| 22 | ES4626F-FLF-38-00079 | Stacking: renumbering and changing the master via front panel button will cause the stack to hang |
| 23 | ES4626F-FLF-38-00050 | Web:It can't remove all(maximum:256) static arp once. |
| 24 | ES4626F-FLF-38-00010 | Logging Sendmail: Mail send too slow (more 3 minutes). |
| 25 | ES4626F-FLF-38-00075 | IPv6AddrCfg: the ping ipv6 command doesn't work for a specific global unicast address |
| 26 | ES4626F-FLF-38-00044 | WebUI: accessing stack members will cause the menu to appear in the logo frame |
| 27 | ES4626F-FLF-38-00061 | L2Switch:It can't remove static mac address from 2th DUT when stacking 2 DUT. |
| 28 | ES4626F-FLF-38-00056 | SNMPv3: An error occurs when DUT snmpwalk to SNMPv2-SMI::mib-2.14.1.6.0. |
| 29 | ES4626F-FLF-38-00071 | IPv6AddrCfg: link-local addresses are not allowed when configuring static neighbor entries via the CLI |
| 30 | ES4626F-FLF-38-00023 | RMON_Auto : etherStatsOversizePkts Value for 1.3.6.1.2.1.16.1.1.1.10.1 is 0 |
| 31 | ES4626F-FLF-38-00070 | IPv6AddrCfg: the command to remove manually configured link local are incorrect |
| 32 | ES4626F-FLF-38-00073 | IPv6AddrCfg: The CLI will display an error message when an illegal static neighbor is configured |
| 33 | ES4626F-FLF-38-00035 | VRRP, Max priority error (255/254) |
| 34 | ES4626F-FLF-38-00065 | 802.1x:The DUT cannot handle the port control when the 802.1x client is used TLS. |
| 35 | ES4626F-FLF-38-00066 | 802.1x:The DUT cannot handle the port control when the 802.1x client is used TTLS. |
| 36 | ES4626F-FLF-38-00067 | 802.1x:The DUT cannot handle the port control when the 802.1x client is used PEAP. |
| 37 | ES4626F-FLF-38-00069 | IPv6AddrCfg: each general prefix name may contain multiple prefix entries |
| 38 | ES4626F-FLF-38-00057 | NetworkMonitor: IfInError.1 can't increase 1000 when injecting 1000 Jabber or oversize packets to port1 by SmartBit. |
| 39 | ES4626F-FLF-38-00058 | NetworkMonitor: Dot3StatsAlignmentErrors.1 can't increase 1000 when injecting 1000 Alignment error packets to port1 by SmartB |
| 40 | ES4626F-FLF-38-00060 | IPv6-MGMT-SNMP:The "etherStatsOverSizePkt" MIB item cannot get any value. |
| 41 | ES4626F-FLF-38-00059 | NetworkMonitor: EtherStatsOversizePkts.1 and EtherStatsJabbers.1 can't increase value when injecting oversize or Jabber packe |
| 42 | ES4626F-FLF-38-00055 | L2Switch: The information of "Flow Control Type" is wrong when DUT enable flow control and Half mode. |
| 43 | ES4626F-FLF-38-00043 | PVLAN:It can't enable pvlan correctly. |
| 44 | ES4626F-FLF-38-00068 | IPv6ACL: ACLs cannot be changed on the fly if more than two ports are bound to it |
| 45 | ES4626F-FLF-38-00082 | Diffserv: the CLI range of values for the meter speed is incorrect |

Runtime V1.1.3.0

| 1 | ES4626F-FLF-38-00052 | QinQ:When the DUT enables the Jumbo frame mode with frame size 1526, the DUT can't forward 1526 bytes untag/ tag packet. |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00083 | Change BUILD_default_MACRO_RAM_HIGH_ADRS from 03B00000 to 0FB00000 |

Runtime V1.2.1.0

| 1 | ES4626F-FLF-38-00080 | IPv6AddrCfg: there is no system enforced maximum number of IPv6 addresses |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00036 | IPv6: DUT shall sets a IsRouter flag true when receiving RA |
| 3 | ES4626H-FLF-17-00266 | New 10G module driver code check in. |
| 4 | ES4626H-FLF-17-00267 | New 10G module web & snmp check in (Helix & Firebolt) |

Runtime V1.2.2.0

| 1 | ES4626F-FLF-38-00019 | IPv6 : Receiving ICMP echo request with hop limit 0, DUT will NOT respond. |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00024 | RMON_Auto : Alarm with Absolute Value ?????Alarm. |
| 3 | ES4626F-FLF-38-00030 | RIP: authentication mode can't be set to MD5 |
| 4 | ES4626F-FLF-38-00034 | [new sdk] IGMP Snooping - Cannot receive querier from low IP address |
| 5 | ES4626F-FLF-38-00038 | When I configured RADIUS server address for IPv6, DUT appeared the error message. |
| 6 | ES4626F-FLF-38-00039 | [new sdk] QinQ : Spanning-tree BPDU. GVRP 不該做 Tunneling |
| 7 | ES4626F-FLF-38-00084 | LLDP:refresh timer is not precise. |
| 8 | ES4626F-FLF-38-00085 | MIB: The value of ifDescr is not correct when some ports belong to some trunk |
| 9 | ES4626F-FLF-38-00086 | ImageFile:Interoperability with different tftp |
| 10 | ES4626F-FLF-38-00087 | ImageFile:Can't upload image file by Web interface from TFTP server |
| 11 | ES4626F-FLF-38-00088 | LACP:admin key test |
| 12 | ES4626F-FLF-38-00089 | LLDP:while jumbo frame disable, maximum frame size TLV of LLDPDU still is 9216 bytes. |
| 13 | ES4626F-FLF-38-00090 | LLDP:show command is abnormal |
| 14 | ES4626F-FLF-38-00092 | ARP:DUT can't learn max arp entries according to the Spec. |
| 15 | ES4626F-FLF-38-00096 | SSHv2:It should refresh to previous page after complete to download user public-key |
| 16 | ES4626F-FLF-38-00097 | LACP:Port Range verify |
| 17 | ES4626F-FLF-38-00098 | VRRP:DUT can't send out packet with VRRP's priority=255 when VR_IP andDUT_IP are different. |
| 18 | ES4626F-FLF-38-00099 | VRRP:DUT must replay arp packets with physical MAC after it disable vrrp function, not vrrp virtual mac |
| 19 | ES4626F-FLF-38-00100 | VRRP:DUT sometime can't learn from arp packets when VRRP function work |
| 20 | ES4626F-FLF-38-00101 | VRRP:It can reply ICMP packets when DUT is not Master |
| 21 | ES4626F-FLF-38-00103 | VRRP:It display no authentication when setting authentication type is simple text by web |
| 22 | ES4626F-FLF-38-00104 | VRRP:Auth Type is empty in web and DUT send out VRRP packets with auth type =255 when setting auth type is no auth by web |
| 23 | ES4626F-FLF-38-00105 | VRRP:From web, it can't set VRRP priority=255 whatever VR_IP and DUT_IP are the same. |
| 24 | ES4626F-FLF-38-00106 | SNMP: OID=1.3.6.1.6.3.18.1.3.0 should not be 0.0.0.0 when DUT send out snmp ipv6 trap packet |
| 25 | ES4626F-FLF-38-00107 | Web:It sometimes can't delete vrrp group entry by web |
| 26 | ES4626F-FLF-38-00109 | Stacking: Device IDs will overlap if a new member is added to replace old members |
| 27 | ES4626F-FLF-38-00110 | LLDP:Port and protocol VLAN ID TLV can't match to configuration |
| 28 | ES4626F-FLF-38-00111 | PortMirror:Destination Mirror port error |
| 29 | ES4626F-FLF-38-00112 | PortMirror:Packet loss |
| 30 | ES4626F-FLF-38-00113 | Can not clear the MAC address of Trunk group in slave unit. (1000 entries) |
| 31 | ES4626F-FLF-38-00114 | RIP:Maximum route |
| 32 | ES4626F-FLF-38-00116 | PVLAN:In stacking mode,while up-link port is trunk port, up-link port can't forward packets to down-link port. |
| 33 | ES4626F-FLF-38-00118 | IGMP Snooping : Query can not forward to another unit through 10GT module. |
| 34 | ES4626F-FLF-38-00119 | 10GT Hot Swap : Slave unit did not show information when insert 10GT module . |
| 35 | ES4626F-FLF-38-00120 | When power off on the unit under cascade environment , the ping command can not ping to the remote unit . |
| 36 | ES4626F-FLF-38-00121 | RPS : Redundant Power Status can not display information when we use RPS to power on the DUT. |
| 37 | ES4626F-FLF-38-00122 | 10G Hot Swap : When remove the 10G module , CLI should display warning message to notice user. |

Runtime V1.2.3.0

| 1 | ES4626F-FLF-38-00132 | Jumbo Frame : Jumbo frame can not forward when we doe all the related jumbo setting from CLI . |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00134 | [Enhancement] Increase username and password length to 32 |

Runtime V1.2.4.0

| 1 | ES4626F-FLF-38-00135 | CLI : System will auto reboot and CLI will display Exception Error Message . |
| --- | --- | --- |

Runtime V1.2.4.6

| 1 | ES4626F-FLF-38-00115 | PVLAN: the configuration of pvlan will overwrite previous configuration. |
| --- | --- | --- |
| 2 | ES4626F-FLF-38-00117 | PVLAN:Can't configure down-link port in different units via CLI. |
| 3 | ES4626F-FLF-38-00129 | Diag wirespeed 10/100/1000 has the same speed, and got strange counter. LED got the same color. |
| 4 | ES4626F-FLF-38-00130 | Diag wirespeed can not test mini Gbic ports. |
| 5 | ES4626F-FLF-38-00136 | [Asante] DUT temperature does not display on WEB. |
| 6 | ES4626F-FLF-38-00137 | [Asante] Display of 'MSTP/Vlan configuration" is very slow |
| 7 | ES4626F-FLF-38-00138 | [Asante] Default router can't be add using WEB UI, always get "fail to set IP address and subnet mask" msg. |
| 8 | ES4626F-FLF-38-00139 | [Asante] Management vlans IP info can't get by using WEB UI |
| 9 | ES4626F-FLF-38-00140 | [Asante] Remote vlan check box is present even if the feature is not supported |
| 10 | ES4626F-FLF-38-00141 | The DUT crashs after config one static MAC address |
| 11 | ES4626F-FLF-38-00142 | There is wrong information on the help command |
| 12 | ES4626F-FLF-38-00143 | RE: Technical problem on ES4650F: (Issue: 16) WEB TFTP |
| 13 | ES4626F-FLF-38-00144 | IPconfig: 192.168.1.255 255.255.0.0 cannot configure |
| 14 | ES4626F-FLF-38-00145 | MIB:No MIB node of"queue weight ***" |
| 15 | ES4626F-FLF-38-00146 | Remove the telnet button from the WEB UI. |
| 16 | ES4626F-FLF-38-00147 | DUT prompt LLDP error message on the console when it connect with the third party switch(Netgear) |
| 17 | ES4626F-FLF-38-00148 | PoE:Port7-24 maximum allocation power range description is wrong. |
| 18 | ES4626F-FLF-38-00149 | Modify the testing speed of diag POST from middle to low speed. |
| 19 | ES4626F-FLF-38-00150 | Need support auto edge and default enable |
| 20 | ES4626F-FLF-38-00151 | Fix SMC ping fail and L3 performance issue |
| 21 | ES4626F-FLF-38-00152 | coming port is wrong while pkt need to trap to cpu port in the end of wirespeed test |

modifications:

Loader V1.1.0.1

| 1. Initial version |  |
| --- | --- |

Loader V1.1.0.2

| 1. Modify UC memory address. (Loader, Diag, Runtime need to change at the same time.) |  |
| --- | --- |

Diag V1.1.0.1

| 1. Initial version |  |
| --- | --- |

Diag V1.1.0.2

| 1. Modify UC memory address. (Loader, Diag, Runtime need to change at the same time.) |  |
| --- | --- |

Runtime V1.1.0.1

| 1. Initial version |  |
| --- | --- |

Runtime V1.1.0.3

| 1. Modify UC memory address. (Loader, Diag, Runtime need to change at the same time.) |  |
| --- | --- |

Runtime V1.2.1.0

| 1. Support EM4626H-XG-10GT |  |
| --- | --- |

Runtime V1.2.3.0

| 1. [Enhancement] Increase username and password length to 32 |  |
| --- | --- |

Known Bugs and Issues:

Any other problems than those listed above should be reported to our Technical Support Staff.

Known RESTRICITIONS AND Limitations:

1. When configuring IPv6 EXT QoS and RATE-LIMIT at same time, the RATE-LIMIT will be not work.

2. QinQ will not support with IGMP snooping at same time.

4. Due to this ASIC do not support “Per port Enable/Disable QinQ”, when issue command “No switchport mode” , the port will actually still remain as “Access mode”.

5. Due to CPU performance, when create maximum vlan number to 4093 vlans, loading/reading configuration file will take more time as usual.

6. As 5., when join all ports with all vlans at the same time, the command will take even more time to complete the job.

7. The MAC addresses learnt at removed units will be either automatically aged out or re-learnt at existed unit ports.

8. Over stacking port monitor will cause receive.

9. Private RIP MIB will not shown when RIP is disabled.

10. Configuration of RIP will be lost when RIP is disabled.

11. (Chip limitation)… When GMII interface is used, the Alignment Error will NOT be detected/reported even there is "alignment error" on the incoming packet . Alignment error will be counted as FCS error if the ingress port is a GMII Interface.

Compliance support:

| Compliance Level | Compliant |
| --- | --- |
| Year 2006 | YES* |

Known Anomalies: None.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |
| IEEE 802.1X | Port-based Network Access Control |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1907 | SNMPv2-MIB (MIB II) | - system group ( 1-9) - snmp group (1-6, 8-22, 24-32) |
| 2011 | IP-MIB (MIB-II) | - ip group (1-20, 22-23) - icmp group (1-26) |
| 2012 | TCP-MIB (MIB-II) | - tcp group (1-15)(tcpConnTable only support read.) |
| 2013 | UDP-MIB (MIB-II) | - udp group (1-5) |
| IEEE 802.1X MIB | IEEE8021-PAE-MIB | - dot1xPaeSystem (1-2) - dot1xPaeAuthenticator (1-3)(not support per-port timer setting) |
| 1493 | Bridge MIB | - dot1dBase group (1-4) - dot1dStp group (1-15) - dot1dTp group (1-4) - dot1dStatic group (1) |
| 2863 | Interfaces Evolution MIB (IF-MIB) | - interface group (1-2) (ifadminStatus not support set to 3(testing)) - ifXTable group (1) - ifStackTable group (1)(support read-only) |
| 2819 | RMON MIB | - statistics group (1) - history group (1-2) - alarm group (1) - event group (1-2) |
| 2618 | RADIUS MIB | - radiusAuthClientMIB (1) |
| 2665 | Etherlike MIB | - dot3StatsTable group (1-19) |
| 2737 | Entity MIB | - entityPhysical group (1-16) (support read-only) |
| 2674 | P-bridge | - dot1dExtBase group (1-4) - dot1dPriority group (1,3) - dot1dGarp group (1) |
| 2674 | Q-bridge | - dot1qBase group (1-5) - dot1qTp group (1-2) - dot1qStatic group(1) - dot1qVlan (1-5) (9-10) |
| V-Bridge MIB | V-Bridge MIB | - dot1vProtocol group |
| 3636 | MAU MIB | - dot3IfMauBasicGroup - dot3IfMauAutoNegGroup |

| 1612 | DNS Reslover MIB | - dnsServConfig - dnsServCounter - dnsServCounterAuthAns - dnsServCounterAuthNoNames - dnsServCounterAuthNoDataResps - dnsServCounterNonAuthDatas - dnsServCounterNonAuthNoDatas - dnsServCounterReferrals - dnsServCounterErrors - dnsServCounterRelNames - dnsServCounterReqRefusals - dnsServCounterReqUnparses - dnsServCounterOtherErrors - dnsServOptCounter - dnsResMIBObjects - dnsResConfig - dnsResCounter - dnsResCache - dnsResCacheStatus - dnsResCacheMaxTTL - dnsResCacheGoodCaches - dnsResCacheBadCaches - dnsResOptCounter |
| --- | --- | --- |
| 3411 | SNMP FrameWork | - snmpFrameworkMIBObjects |
| 3412 | SNMP MPD MIB | - snmpMPDMIBObjects |
| 3413 | SNMP Target MIB | - snmpTargetObjects |
| 3413 | SNMP Notify MIB | - SnmpNotifyObjects - snmpNotifyTable - usmMIBObjects |
| 3415 | SNMP View-Based ACM MIB | - vacmMIBObjects - Private MIB - switchMgt - portMgt - trunkMgt - lacpMgt - staMgt - tftpMgt - restartMgt - mirrorMgt - igmpSnoopMgt - ipMgt - bcastStormMgt - vlanMgt - priorityMgt - trapDestMgt - qosMgt - securityMgt - sysLogMgt - lineMgt |

ES4626H/ES4650HD Private Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| SMC8824_48M.mib | 20050726001 |

SNMP Trap Support:

| RFC No. | Title |
| --- | --- |
| RFC 1215 | ColdStart (v1) warmStart (v1) linkDown (v1) linkUp (v1) authenticationFailure (v1) |
| RFC 1907 | coldStart (v2c) warmStart (v2c) authenticationFailure (v2c) |
| RFC 2863 | linkDown (v2c) linkUp (v2c) |
| RFC 1493 | newRoot (v1) topologyChange (v1) |
| RFC 1757 | risingAlarm (v1) fallingAlarm (v1) |
| RFC 2819 | risingAlarm (v2c) fallingAlarm (v2c) |

Customer Support:

By Phone:

By Email:

By Web:

By Fax: