SMC8126L2(SMC8150L2)

Firmware Version 1.4.3.3

INTRODUCTION:

SMC8126L2(SMC8150L2) is a switch containing defines 26-port(50-port) Gigabit Ethernet Layer 2/4 Switch. The switch provides 26(50) 10/100/1000GbE CU ports.  Four of those ports are combo ports with associated SFP transceiver slots21~24(45~48).

The switch can be managed either by in-band management via the network station remotely or out-of-band management via the console port (RS232) locally. An imbedded Web agent also provides management capability to any computer on the network via common Http browsers such as Netscape Navigator or Microsoft’s Internet Explorer (both browsers should be Version 6.0 or above).

Local Console Management (LCM) allows the user to monitor and configure the SMC8126L2(SMC8150L2) from a VT-type terminal. LCM can be used to configure features such as SNMP community names and access rights, Port Enable/Disable, firmware downloads, and Device IP address as well as most other parameters. LCM can also provide statistical and diagnostic information about the entire device or an individual port.

Management of the switch is password protected; the same password is used for LCM and for the Web browser interface. Prior to accessing the Management Module via a network connection, a valid IP address, subnet mask, and in some cases a default gateway must be configured using an out-of-band connection or the BootP/DHCP protocol. The management option provides SNMP, RMON (4 groups: 1,2,3,9), and Web management for system control and statistical monitoring.

Firmware Specification:

| Status | Version No. | Type | Release Date |
| --- | --- | --- | --- |
| Patch | 1.4.3.3 | Customer | 2011/9/16 |
| Patch | 1.4.3.2 | Customer | 2011/9/13 |
| Patch | 1.4.3.0 | Customer | 2011/6/8 |
| Patch | 1.4.2.3 | Customer | 2011/5/6 |
| Patch | 1.4.2.2 | Customer | 2011/2/21 |
| Patch | 1.4.2.0 | Customer | 2010/12/2 |
| Patch | 1.4.1.1 | Customer | 2010/11/5 |
| Patch | 1.4.1.0 | Customer | 2010/10/28 |

SUPPORTED FUNCTIONALITY:

| Category | Feature | Support |
| --- | --- | --- |
| Layer 2 Features |  |  |
| Link Aggregation | 802.3ad with LACP | Yes |
|  | Cisco EtherChannel Liked | Yes |
|  | Unicast / Multicast traffic Balance over Trunking port | Yes |
| VLAN | IEEE 802.1Q Tagged Based | Yes |
|  | Port-Based | Yes |
|  | Private VLAN(only Traffic segmentation) | Yes |
|  | GVRP | Yes |
|  | 802.1v Protocol | Yes |
|  | Community VLAN | Yes |
|  | Voice VLAN | Yes |
| Spanning Tree | IEEE 802.1D | Yes |
|  | IEEE 802.1s | Yes |
|  | IEEE 802.1w | Yes |
|  | Spanning Tree Fast Forwarding | Yes |
| BroadCast Strom Control |  | Yes |
| Multicast Strom Control |  | Yes |
| Unknown Unicast Strom Control |  | Yes |
| IGMP Snooping | IGMP Snooping v1/v2 | Yes |
|  | IGMP v1/v2 querier support | Yes |
|  | IGMP Immediate Leave | Yes |
|  | IGMP throttling | Yes |
|  | IGMP Filtering | Yes |
|  | IFMPSNP Leave Proxy | Yes |
|  | IGMPV3 Snooping | Yes |
| QinQ |  | Yes |
| MVR |  | Yes |
| Jumbo Frame | 9K | Yes |
| QoS Features |  |  |
| Scheduling for pripority queue | WRR Pripority scheduling | Yes |
|  | Strict Pripority scheduling | Yes |
|  | Hybrid (WRR + Strict) Pripority scheduling | No |
| COS | 802.1p based COS | Yes |
|  | IP TOS/Precedence based COS | Yes |
|  | DSCP based COS | Yes |
|  | TCP/UDP Port based COS | Yes |
| Rate Limiting | Ingress | Yes |
|  | Egress | Yes |
| DiffServ |  | Yes |
| Security Features |  |  |
| Static Port Security (MAC-based) |  | Yes |
| 802.1x | Port-Based | Yes |
|  | Mac-Based | Yes |
|  | Supplicant Support | Yes |
|  | QoS assignment | Yes |
|  | Guest VLAN | Yes |
|  | EAPOL frames pass-through | Yes |
| ACL | L2/L3/L4 | Yes |
| RADIUS authentication |  | Yes |
| TACACS+ Authentication |  | Yes |
| TACACS+ 3.0 |  | Yes |
| MAC Based Authentication |  | Yes |
| HTTPS and SSL (Secured Web) |  | Yes |
| SSH V2.0 (Secured Telnet session) |  | Yes |
| User name password authentication | Local Authentication | Yes |
|  | Remote Authentication via RADIUS/TACACS+ | Yes |
| Management Interface Access Filtering | SNMP | Yes |
|  | WEB | Yes |
|  | TELNET | Yes |
| Management Features |  |  |
| Cisco Like CLI |  | Yes |
| Web Based Management |  | Yes |
| Telnet |  | Yes |
| Software Download/ upgrade | TFTP | Yes |
|  | Xmodem | Yes |
| Dual Image |  | Yes |
| Configuration download/ upload | TFTP | Yes |
| SNMP | v1 | Yes |
|  | v2c | Yes |
|  | v3 | Yes |
| RMON | RMON I (1,2,3,9 group) | Yes |
| BOOTP | Client | Yes |
| DHCP | Client | Yes |
|  | Option 82 Relay | Yes |
|  | Snooping | Yes |
| Port Mirroring |  | Yes |
| RSPAN |  | Yes |
| Event/Error Log | Local Flash | Yes |
|  | Remote server via System Log (RFC3164) | Yes |
|  | SMTP | Yes |
| DNS | Client | Yes |
|  | Proxy | Yes |
| Remote Ping |  | Yes |
| SNTP |  | Yes |
| IP Clustering |  | Yes |
| IP Source Guard |  | Yes |
| LLDP |  | Yes |
| Delay reload |  | Yes |
| Press [A] to continue display |  | Yes |
| Show interfaces Brief |  | Yes |
| Banner message |  | Yes |
| length of user name and password extend to 32 |  | Yes |

Installation and Configuration Notes:

In general, the SMC8126L2(SMC8150L2) will be shipped to you pre-configured with this version of firmware. If you would like to upgrade an existing SMC8126L2(SMC8150L2), please follow the TFTP download instructions that are included with your firmware image upgrade kit. TFTP download instructions are also available on the Support web site.

Firmware Changes and Enhancements:

V1.4.3.3

1. Fails to configure auto dhcp configure boot up setting on SMC8126L2

2. After rebooting, the console hang up.

V1.4.3.2

1. ES4526R-FLF-17-LN-00001 [Request] SMC8150L2 function enhancement --- DHCP Auto Configuration

V1.4.3.0

1. ES4526R-FLF-17-00197: CLI show more than one page,need to provide guideline! (rollback)

V1.4.2.3

1. ES4526R-FLF-17-00198: Cannot add a port as tagged with web UI.

2. ES4526R-FLF-17-00199: lldp protocol block the port of the SMC8126L2

V1.4.2.2

1. broadcast storm packet rate minimum value from 500 to 64

V1.4.2.0

1. ES4526R-FLF-17-00190: dot1x can't work with PEAP authenticate

2. support DHCP snooping option 82 Remote ID/Circuit ID configurable

V1.4.1.1

1. ES4550R-FLF-17-00008: MVR: add mvr group range configuration

V1.4.1.0

- ES4526R-FLF-17-00184: SMC8126L2 doesn't send out security trap after set static mac-address.

V1.4.0.2

1 ES4526R-FLF-17-00108: 802.1x:It doesn't display the port info..

- ES4526R-FLF-17-00125: CLI:The description of error message about RSPAN should express obviously.

- ES4526R-FLF-17-00129: RSPAN:The configuration of RSPAN VLAN will loss after saving and rebooting

- ES4526R-FLF-17-00159: Switch will learn switch own mac into arp table.

- ES4526R-FLF-17-00168: SNMP:It can't allow manager set up to 5 trap receiver..

- ES4526R-FLF-17-00169: T DNS:The second entry should fail to set or overwrite when setting duplicate ip host address but different host name.

- ES4526R-FLF-17-00171: Cluster: The decription of the member disappear after rebooting.

- ES4526R-FLF-17-00172: Cluster:It display "candidate" in web When The role is "member to be" in CLI

- ES4526R-FLF-17-00173: Cluster: It must not to set commander and ip-pool when enable cluster by web

- ES4526R-FLF-17-00174: Cluster: It should display default ip pool address when enable commander.

- ES4526R-FLF-17-00176: DynamicQos:DUT can not keep Dynamic QoS Assignment settings with copying running to startup.

- ES4526R-FLF-17-00177: LLDP: The LLDP Data Unit in the frame which is sent by DUT is incorrect.

- ES4526R-FLF-17-00178: L2Switch:Flow control can't work.

- ES4526R-FLF-17-00179: All the entire entry of the dot1qstaticunicaststatus and dot1qstaticunicastallowedtogoto

V1.4.0.1

Following are fixed in this version

- ES4526R-FLF-17-00061: L2Switch:We can't set flow control function on the WebUI.

- ES4526R-FLF-17-00069: DNS: After replace the hostname setting, DUT can not show the correct configure.

- ES4526R-FLF-17-00074: IP source guard:Can not configure static ip source guard binding entry on each port in Web UI

- ES4526R-FLF-17-00075: Cluster: Web and CLI display will show wrong members when static member over 9.

- ES4526R-FLF-17-00077:L2 switch- No place be found from WEB to configure flow control.

- ES4526R-FLF-17-00140: Trunk:Create a static trunk,commmand "sh run" ,it displays "lacp" word in running config.

- ES4526R-FLF-17-00141:CLI:no any port or trunk can join RSPAN VLAN, so the column"Ports/Port Channels:" should be removed"

V1.4.0.0

Following new features have been supported from this version

- Voice VLAN

- QoS assignment

- Guest VLAN

- EAPOL frames pass-through

V1.3.0.2

Following new features have been supported from this version

- Loop back detection

- Auto edge

- LLDP

- DHCP snooping

- IP source guard

- Delayed reload

- Show ARP(and clear)

- Press [A] to continue Display

- Show interfaces Brief

- Banner message

- length of user name and password extend to 32

Known ISSUES:

V1.0.0.8

- IP source guard:Can not configure static ip source guard binding entry on each port in Web UI

- Cluster: Web and CLI display will show wrong members when static member over 9.

- ProtocolVLAN:It cannot set protocol group and VLAN mapping on page "Protocol VLAN Port Configuration" on Firefox browser.

- L2 switch- No place be found from WEB to configure flow control.

- DiffServ:It cannot set cos value of policy map on Web browser(page "policy rule settings").

Known Restrictions and Limitations:

- Due to small and limited packet buffer of switch ASIC, when the jumbo frame bigger than 7k will lose packets on one 1000M/F to ten 100M/F and one 100M/F to ten 10M/F

- Due to limited number of filter processor in ASIC. The feature IP Source Guard and Diffserv for IP packet can’t be enabled at the same time. Once user enabled one of them, the other one would be unable to configure till user has disabled the first feature.

- Because chip do not support “per port rate-limit” function, so scheduling with egress rate limit can’t work correctly on 8126/50L2

Any other problems than those listed above should be reported to our Technical Support Staff.

IEEE Standards Support:

| Standard | Title |
| --- | --- |
| IEEE 802.1D | Transparent Bridging Specifications (ISO/IEC 10038) |
| IEEE 802.1p | Traffic Class Expediting and Dynamic Multicast Filtering |
| IEEE 802.1Q | Virtual Bridged Local Area Networks |
| IEEE 802.1v | Protocol VLAN |
| IEEE 802.1w | Rapid Spanning Tree |
| IEEE 802.1s | Multiply Spanning Tree |
| IEEE 802.2 | Local Area Networks, Logical Link Control (LLC) |
| IEEE 802.3 | CSMA/CD 9 (ISO/IEC 8802-3) |
| IEEE 802.3I | 10Base-T (ISO/IEC 8802-3, clause 14) |
| IEEE 802.3u | 100Base-TX (ISO/IEC 8802-3, clause 25) |
| IEEE 802.3x | Flow Control |
| IEEE 802.3z | 1000Base-SX, 1000Base-LX |
| IEEE 802.3ab | 1000Base-TX |
| IEEE 802.3ac | VLAN Tag |
| IEEE 802.3ad | Link Aggregation |

IETF Standards MIB Support:

| RFC No. | # Title | Groups Supported |
| --- | --- | --- |
| 1213 | MIB-II | - system group - interfaces group - ip group - icmp group - tcp group - udp group - snmp group |
| 1493 | Bridge MIB | - dot1dBase group - dot1dStp group - dot1dTp group - dot1dStatic group |
| 1907 | SNMPv2 MIB |  |
| 2011 | IP MIB |  |
| 2012 | TCP MIB |  |
| 2013 | UDP MIB |  |
| 2576 | SNMP COMMUNITY MIB |  |
| 2618 | RADIUS AUTH CLIENT MIB |  |
| 2665 | EtherLike-MIB |  |
| 2863 | Interfaces Evolution MIB | - ifXTable group - ifStackTable group |
| 2819 | RMON MIB | - statistics group - history group - alarm group - event group |
| 2618 | RADIUS MIB | - radiusAuthClientMIB |
| 2665 | Etherlike MIB | - dot3StatsTable group |
| 2737 | Entity MIB | - entityPhysical group |
| 2674 | P-bridge | - dot1dExtBase group - dot1dPriority group - dot1dGarp group |
| 2674 | Q-bridge | - dot1qBase group - dot1qTp group - dot1qStatic group - dot1qVlan |
| 2668 | MAU MIB | - dot3IfMauBasic group - dot3IfAutoNeg group |

Private Enterprise MIB Support:

| Title | #### Version |
| --- | --- |
| SMC8126_50R.mib |  |