OAP6626A-FLF-EC Enterprise Wireless Access Point

Firmware Release Notes

Version 4.3.7.2z

Release Date: Nov. 16, 2010

Introduction:

Firmware Specification:

| Status | Version No. | Release Date | # History |
| --- | --- | --- | --- |
| Special Release (release to Edgecore Europe customer) | 4.3.7.2z | 11/16/2011 | Recover country Z9 and SNTP |
| Formal Release (release to NTC & Edgecore 20110329) | 4.3.7.2 | 1/11/2011 | Fix all NTC bugs (7 bugs) |
| Formal Release (release to NTC) | 4.3.7.1 | 12/31/2010 | Fix all NTC bugs (18 bugs) and specific MAC issue |
| Formal Release (release to NTC) | 4.3.4.8 | 10/22/2010 | Fix NTC bugs and add a 19.5 dBi external antenna (4.9GHz) |
| Formal Release (release to NTC) | 4.3.4.7 | 10/11/2010 | Initial release |

v4.3.7.2z Changes:

- Based on v4.3.7.2, to recover special country Z9 setting(no DFS detection) and SNTP on v4.3.4.3:

  - modify define OEM_ADPT_NON_DFS_COUNTRY_SUPPORT to 1 (support country Z9):

oem_adpt.h

v4.3.4.3 Changes and Fixed bugs:

Problem: Base on customer requirement, we provide a non-exist country Z9 to avoid DFS effect.

Solution: We increase non-exist regulation domain ETSID, and non-exist country Z9 to support this requirement.

v4.3.7.2 Changes:

- Fix all NTC bugs (7 bugs: 4 new bugs, 3 re-open bugs):

  - OAP6626A-FLF-EC-00009:

When user press “Logout” on advanced page, AP will close the current window and terminate this session.

  - OAP6626A-FLF-EC-00014:

Root-cause: To press “Enter” key will bypass javascript check function.

We add the javascript function in form of html: onSubmit=”return ssh_save()”  to enter our javascript check function forcely.

  - OAP6626A-FLF-EC-00021:

This is not a real bug. Because the NTC tester input “a.a.a.a” as host name of a syslog server in local LAN, this host name will be sent to FQDN for checking. Because there is no any DNS server in tester’s test environment, the response will time-out (Ex: wait 2-3 minutes).

  - OAP6626A-FLF-EC-00025:

This root-cause is the same as OAP6626A-FLF-EC-00028.

When changing "Root" to "Master", user should do some setting's modifications as follows.

1. Copy original peer's (Slave) MAC to Bridge Child (one of 2~6).

2. Then input “00-00-00-00-00-00” to Bridge parent.

  - OAP6626A-FLF-EC-00026:

Root-cause: There is hard code to passphrase for PSK key type in auto-provision mode.

  - OAP6626A-FLF-EC-00027:

Original WDS design doesn’t support Root-Master link case.

  - OAP6626A-FLF-EC-00028:

Root-cause: There is hard code to Open-System on WDS setting page when changing WDS role in auto-provision mode.

v4.3.7.1 Changes:

- Fix all NTC bugs (18 bugs):

  - OAP6626A-FLF-EC-00014:

modify design : limit SSH port range as 1025 ~ 65535 and default is 22

  - OAP6626A-FLF-EC-00024:

add a config setting to sync. security on Web page of WDS.

When auto provision is enabled, if user input correct Master’s MAC and SSID in Slave mode, Master’s security setting will apply to Slave automatically.

  - OAP6626A-FLF-EC-00022:

modify security page for each vap to simplify setting.

When user select auth type, AP will select correct a set of encryption and 802.1X automatically.

However, because open-system can combine WEP or None encryption to achieve static WEP, and enable/disable 802.1X to achieve dynamic WEP, AP won’t help select encryption and 802.1X automatically.

- Client with specific Ethernet MAC (54:42:49:ED:C3:F5) can’t access through WDS + encryption (WPA-PSK)   merge solution from Aruba project (AP-80).

v4.3.4.8 Changes:

- Fix NTC bugs:

  - OAP6626A-FLF-EC-00002

  - OAP6626A-FLF-EC-00005 (ch: 184, 188, 192, 196, 8, 12, 16)

  - OAP6626A-FLF-EC-00006 (ch: 184, 188, 192, 196, 8, 12, 16)

- Add antennas and hide unused antennas (not support 4.9GHz)

  - add integrated (embedded 14 dBi)

  - add external antenna (Mitsubishi ACF-050GU-A1 19.5 dBi High-Gain Directional Panel Ant)

- Modify MAC’s assigned rule of wireless interface

v4.3.4.7 Changes:

- Inherit version v4.3.6.5 of OAP6626A-FLF-17 main branch codebase and modify some compiling issues

- 4.9GHz radio band support in J3 country code

- Edge-Core brand support

- Merge bugs’ solution of OAP6626A-FLF-02 in v5.1.0.9 NTC reported bugs (5 Critical & 11 Major) :

  - Bug lists are attached on CQ and local file server. (except for OAP6626A-FLF-02-00010)

System Default value:

| Identification |  |  | Default Value |
| --- | --- | --- | --- |
|  |  | sysname | Enterprise Wireless AP |
| IP |  |  |  |
|  |  | DHCP | Enable |
|  |  | Ipaddress | 192.168.1.1 |
|  |  | subnetmask | 255.255.255.0 |
|  |  | default gate way | 0.0.0.0 |
|  |  | pri DNS address | 0.0.0.0 |
|  |  | sec DNS address | 0.0.0.0 |
| RADIUS |  |  |  |
|  | Primary |  |  |
|  |  | IP/ host name | 0.0.0.0 |
|  |  | port | 1812 |
|  |  | key |  |
|  |  | timeout | 5sec |
|  |  | retransmit | 3 |
|  | Secondary |  |  |
|  |  | IP/ host name | 0.0.0.0 |
|  |  | port | 1812 |
|  |  | key |  |
|  |  | timeout | 5sec |
|  |  | retransmit | 3 |
| Authentication |  |  |  |
|  | MAC |  |  |
|  |  | MAC | Disable |
|  | Local MAC authentication |  |  |
|  |  | system default | Allowed |
|  |  | permission |  |
|  |  | MAC |  |
|  | 802.1x |  |  |
|  |  | 802.1x | Disable |
| Filter Control |  |  |  |
|  |  | localbridge | Disable |
|  |  | AP manager | Enable |
|  |  | ehernet type | Disable |
| SNMP |  |  |  |
|  |  | location |  |
|  |  | contact | Contact |
|  |  | community(read only) | public |
|  |  | community(read/write) | private |
|  |  | trap destination IP 1 | 0.0.0.0 |
|  |  | trap community | public |
| Administrator |  |  |  |
|  |  | username | admin |
|  |  | password | <NULL> |
|  |  | firmware filename | zz-img.bin |
|  |  | ftp/tftp user name |  |
|  |  | ftp/tftp user password |  |
| System Logging |  |  |  |
|  |  | syslog | Disable |
|  |  | logging host | Disable |
|  |  | IP/ server name | 0.0.0.0 |
|  |  | logging console | Disable |
|  |  | logging level | Informational |
| Radio a |  |  |  |
|  |  | channel | There's no 11a channel supported by this country. |
|  |  | auto ch select | Enable |
|  |  | antenna ID | id=0x0000, module=NA |
|  |  | antenna location | Outdoor |
|  |  | transmit power | 100% |
|  |  | maximum supported rate | 54 Mbps |
|  |  | beacon interval | 100 TUs |
|  |  | DTIM | 1 Beacons |
|  |  | RTS | 2347 Bytes |
| Radio a security |  |  |  |
|  |  | SSID 0 (Radio a) | VAP_TEST_11A 0 |
|  |  | SSID 1 (Radio a) | VAP_TEST_11A 1 |
|  |  | SSID 2 (Radio a) | VAP_TEST_11A 2 |
|  |  | SSID 3 (Radio a) | VAP_TEST_11A 3 |
|  |  | auth type | Open system |
|  |  | Multicast-cipher | WEP |
|  |  | Unicast-cipher | TKIP and AES |
|  |  | WPA client | Disabled |
|  |  | Encryption | Disabled |
|  |  | key length | None |
|  |  | key type | Hexadecimal |
|  |  | Key num | 1 |
|  |  | key |  |
| Radio g |  |  |  |
|  |  | channel | There's no 11g channel supported by this country. |
|  |  | auto ch select | Enable |
|  |  | antenna ID | id=0x0000, module=NA |
|  |  | antenna location | Outdoor |
|  |  | transmit power | 100% |
|  |  | maximum supported rate | 54 Mbps |
|  |  | beacon interval | 100 TUs |
|  |  | DTIM | 1 Beacons |
|  |  | RTS | 2347 Bytes |
| Radio g security |  |  |  |
|  |  | SSID 0 (Radio g) | VAP_TEST_11G 0 |
|  |  | SSID 1 (Radio g) | VAP_TEST_11G 1 |
|  |  | SSID 2 (Radio g) | VAP_TEST_11G 2 |
|  |  | SSID 3 (Radio g) | VAP_TEST_11G 3 |
|  |  | auth type | Open system |
|  |  | Multicast-cipher | WEP |
|  |  | Unicast-cipher | TKIP and AES |
|  |  | WPA client | Disabled |
|  |  | Encryption | Disabled |
|  |  | key length | None |
|  |  | key type | Hexadecimal |
|  |  | Key num | 1 |
|  |  | key |  |

Known issues: